# Choice of Topology and Transmission Medium in LAN Design

## I. Choice of Topology

The network topology refers to the physical and logical layout of a network. The choice of topology in a LAN (Local Area Network) impacts its reliability, performance, expandability, and maintenance.

This choice is closely tied to:

- The transmission medium

- The wiring layout

- The access control technique

Four major types of media have been used for bus LANs:

## 1. Twisted Pair

- Type: Voice-grade (unshielded)

- Use: Early LANs (1 Mbps)

- Advantages: Inexpensive, easy to install

- Disadvantages: Cannot scale to high data rates in bus configuration

- Status: Obsolete for shared bus LANs

## 2. Baseband Coaxial Cable

- Type: Digital signaling

- Used In: Original Ethernet

- Advantages: Supported wide usage historically

- Disadvantages: Difficult to modify; requires physical changes

- Status: Legacy systems still exist, but no new installations

## 3. Broadband Coaxial Cable

- Type: Analog signaling (like cable TV)

- Advantages: High capacity

- Disadvantages: Complex, costly, hard to maintain

- Status: Never popular for LANs; obsolete

## 4. Optical Fiber

- Advantages: High bandwidth, electromagnetic isolation, compact

- Disadvantages: Expensive, difficult to tap in bus topology

- Status: Not used for bus LANs; better for other topologies

## II. Topology Comparisons

Ring Topology:

- Best throughput potential; suitable for long distances

- Drawback: A single failure can disable the network

Star Topology:

- Simple and natural for buildings; centralized hub/switch

- Suitable for short distances and high-speed connections

## III. Choice of Transmission Medium

The transmission medium impacts performance, reliability, and scalability. Influencing factors include:

- Capacity

- Reliability

- Types of data supported

- Environmental scope

## Common Transmission Media in LANs

1. Voice-grade UTP (Category 3): Low cost, limited data rate, obsolete

2. Shielded Twisted Pair & Baseband Coaxial: Higher capacity, moderate cost

3. Broadband Coaxial: High capacity, complex and costly

4. High-Performance UTP (Category 5): Popular choice; supports star topology

5. Optical Fiber: High performance but expensive and needs expertise

## IV. Conclusion

Modern LANs prefer star topologies using Category 5 UTP or optical fiber.

Bus topologies and coaxial cables are largely obsolete.