# Comprehensive Notes on LAN Protocol Architecture

# 1. Introduction to LAN Protocol Architecture

Local Area Networks (LANs) require a protocol architecture that enables reliable communication between interconnected devices.
This architecture is layered and typically follows the IEEE 802 standards, which closely align with the lower layers of the OSI model.
LAN protocol architecture includes the Physical Layer, Medium Access Control (MAC) Layer, and Logical Link Control (LLC) Layer.
Each layer is responsible for a specific set of functions that ensure data is transmitted accurately and efficiently across the network.

# 2. IEEE 802 Reference Model

The IEEE 802 reference model refines the OSI model’s data link layer into two sublayers—LLC and MAC—and retains the physical layer.
These three layers in the IEEE 802 LAN architecture are:
• Logical Link Control (LLC): Manages communication between the data link layer and network layer, handles error checking, and flow control.
• Medium Access Control (MAC): Governs access to the physical medium and provides addressing and channel access mechanisms.
• Physical Layer: Handles transmission and reception of raw bit streams over a physical medium.

# 3. Logical Link Control (LLC)

The LLC sublayer is defined in IEEE 802.2 and provides an interface for the upper layers while managing data link control functions.
Key Features:
• Supports multiple network protocols (e.g., IP, IPX)
• Provides multiplexing capability using Service Access Points (SAPs)
• Offers three types of communication services:
  1. Unacknowledged Connectionless Service: For high-speed transfers with minimal error handling.
  2. Connection-Mode Service: Includes sequence control, acknowledgments, and error control.
  3. Acknowledged Connectionless Service: For delivery confirmation without a full connection setup.
LLC Protocol Data Units (PDUs) include control fields and addressing via DSAP and SSAP. Common control commands include SABME (set asynchronous balanced mode extended), DISC (disconnect), UA (unnumbered acknowledgment), and XID/TEST for management purposes.

# 4. Medium Access Control (MAC)

The MAC sublayer is responsible for controlling how devices gain access to the medium and permission to transmit data.
Functions include:
• Frame delimiting and recognition
• Addressing using MAC addresses
• Error detection (usually CRC)
• Coordination of shared medium access

Access Mechanisms:
• Contention-Based (e.g., CSMA/CD used in Ethernet): Devices compete for medium.
• Token-Based (e.g., Token Ring): Access given via passing a token.
• Scheduled Access (e.g., polling or reservation systems): Suitable for time-sensitive applications.

MAC Frame Structure:
• MAC Header: Includes destination and source MAC addresses.
• MAC Control: Indicates frame type and control information.
• Payload: Contains data from LLC.
• Frame Check Sequence (FCS): Provides error detection.

# 5. Physical Layer

The physical layer of a LAN defines the electrical and physical specifications for devices. It includes:
• Encoding techniques (e.g., Manchester encoding)
• Bit transmission over media (copper, fiber)
• Connector types and cable standards (e.g., RJ45, ST connectors)
• Transmission rates and signaling methods
Examples:
• 10BASE-T: Uses unshielded twisted pair (UTP), 10 Mbps
• 100BASE-FX: Uses fiber-optic cable, 100 Mbps
• Gigabit Ethernet: Supports up to 1 Gbps using Category 5e or fiber

# 6. Summary

LAN protocol architecture is essential for structured and efficient communication. It ensures reliable data delivery across a shared medium.
The IEEE 802 reference model—comprising LLC, MAC, and Physical Layers—enables interoperability, scalability, and robust network design.
A clear understanding of each layer's responsibilities helps in designing and troubleshooting LANs effectively.