# IEEE 802.3 Medium Access Control (MAC)

## 1. Introduction

IEEE 802.3 is the standard for Ethernet. Its Medium Access Control (MAC) sublayer is responsible for managing access to the shared transmission medium.

## 2. Precursors to CSMA/CD: Random Access Techniques

### a. Random Access / Contention Techniques

These techniques allow transmissions without a predetermined schedule. Stations transmit randomly and contend for the medium, which may lead to collisions.

### b. Pure ALOHA

Developed for packet radio networks. Stations transmit anytime and wait for an acknowledgment (ACK). If no ACK is received, the station retransmits. Collisions are common. Utilization is about 18%.

### c. Slotted ALOHA

Time is divided into slots. Transmission begins only at the start of a slot. Utilization improves to about 37%.

### d. Limitations of ALOHA

High collision rates and poor channel efficiency. Does not leverage the short propagation delay in LANs.

## 3. Carrier Sense Multiple Access (CSMA)

### a. Motivation

LANs typically have small propagation delay. Stations can detect channel usage quickly, minimizing collisions.

### b. Basic Principle

1. Sense the medium. If idle, transmit.
2. If busy, wait and retry.
3. Wait for ACK after transmission.
4. If no ACK, assume collision and retransmit.

### c. Advantages

More efficient than ALOHA. Utilization improves with longer frames or shorter propagation delays.

## 4. CSMA Variants (Persistence Strategies)

### a. Nonpersistent CSMA

1. Sense the channel. If idle, transmit.
2. If busy, wait a random time before retrying.

Reduces collision risk but may leave channel idle unnecessarily.

### b. 1-Persistent CSMA

1. If idle, transmit immediately.
2. If busy, wait until idle and transmit immediately.

Efficient at low load but leads to collisions when multiple stations wait.

### c. P-Persistent CSMA

1. If idle, transmit with probability p, or wait one time slot with 1-p.
2. If busy, wait until idle then repeat step 1.
3. After delay, repeat step 1.

Helps balance collisions and idle time. Proper tuning of p is critical.

## 5. Summary of Persistence Strategies

| Strategy | Idle Medium Action | Busy Medium Action | Pros | Cons |
| --- | --- | --- | --- | --- |
| Nonpersistent | Transmit | Wait random time, retry | Fewer collisions | Wastes channel time |
| 1-Persistent | Transmit immediately | Wait until idle, then transmit | Efficient at low loads | High collision probability |
| P-Persistent | Transmit with probability p | Wait until idle, repeat with p | Balances delay and collisions | Requires tuning of p |

## 6. CSMA/CD: Carrier Sense Multiple Access with Collision Detection

Used in IEEE 802.3 (Ethernet). Stations detect collisions during transmission, send a jam signal, and back off for a random period before retrying.

## 7. Efficiency and Performance

Utilization improves with longer frames and shorter propagation delays. Trade-offs in persistence algorithms must be managed to avoid network overload and instability.

## 8. Persistence Strategies

[Nonpersistent CSMA] - Wait random time if busy
[1-Persistent CSMA] - Transmit as soon as channel is idle
[P-Persistent CSMA] - Transmit with probability p; delay with probability (1-p)

## 9. Ethernet Frame Preamble: 7-Octet Pattern

In IEEE 802.3 (Ethernet), every frame starts with a preamble to allow receivers to synchronize with the sender’s clock before actual data transmission begins.

### a. Structure of Preamble and Start Frame Delimiter (SFD)

| Field | Size | Pattern |
| --- | --- | --- |
| Preamble | 7 Octets (56 bits) | 10101010 (repeated 7 times) |

| Field | Size | Pattern |
| --- | --- | --- |
| Start Frame Delimiter (SFD) | 1 Octet (8 bits) | 10101011 |

The SFD marks the end of the preamble and signals the start of the actual frame data.

### b. Visual Representation of Ethernet Frame Beginning

[Preamble] [SFD] [Destination Address] [Source Address] [Type/Length] [Payload] [FCS]
  7 bytes   1 byte       6 bytes            6 bytes       2 bytes     46–1500B   4 bytes

## Purpose

## Helps synchronize sender and receiver clocks.

## Indicates that a valid Ethernet frame is about to begin.

## Ensures timing accuracy for decoding the data bits that follow

## 10. IEEE 802.3 MAC Frame Format

The IEEE 802.3 MAC frame format consists of several fields as depicted in Figure 16.3. Each field plays a distinct role in delivering and validating Ethernet frames.

### Preamble

A 7-octet pattern of alternating 0s and 1s used by the receiver to establish bit synchronization.

### Start Frame Delimiter (SFD)

The sequence 10101011 indicating the start of the frame and allowing the receiver to identify the beginning of the actual frame data.

### Destination Address (DA)

Specifies the intended receiver(s) of the frame. It can be a unique physical address, group address, or broadcast (global) address.

### Source Address (SA)

Specifies the originating station that sent the frame.

### Length/Type

Indicates either the length of the LLC data (if IEEE 802.3) or the Ethernet type field (if Ethernet II format). Maximum frame size (excluding preamble and SFD) is 1518 octets.

### LLC Data

Contains the data unit supplied by the Logical Link Control (LLC) sublayer.

### Pad

Additional octets added if needed to ensure the minimum frame size required for collision detection.

### Frame Check Sequence (FCS)

A 32-bit CRC used for error detection. It covers all fields except the preamble, SFD, and the FCS itself.

### a. Visual Representation of IEEE 802.3 MAC Frame Format

[Preamble] [SFD] [DA] [SA] [Length/Type] [LLC Data + Pad] [FCS]
  7 bytes   1 byte  6 bytes 6 bytes    2 bytes     46–1500 bytes    4 bytes

SFD = Start Frame Delimiter
DA = Destination Address
SA = Source Address
FCS = Frame Check Sequence