# IEEE 802.3 Ethernet Standards – Teaching Notes

## 1. IEEE 802.3 10-Mbps Ethernet (Standard Ethernet)

Overview:

- First Ethernet standard based on CSMA/CD

- Operates at 10 Mbps

- Defined under IEEE 802.3

Features:

| Feature | Description |
| --- | --- |
| Data Rate | 10 Mbps |
| Topology | Originally bus, later star |
| Medium | Coaxial cable, UTP, or fiber |
| Access Method | CSMA/CD |
| Frame Format | IEEE 802.3 |

Common Physical Layer Standards:

| Standard | Medium | Max Length | Notes |
| --- | --- | --- | --- |
| 10BASE5 | Thick coaxial | 500 m | Original Ethernet (backbone) |
| 10BASE2 | Thin coaxial | 185 m | Cheaper, flexible |
| 10BASE-T | Twisted pair (UTP) | 100 m | Star topology, widely used |
| 10BASE-F | Fiber optic | 2000 m | Long-distance connections |

## 2. IEEE 802.3u – 100 Mbps Ethernet (Fast Ethernet)

Overview:

- Upgrade of standard Ethernet to 100 Mbps

- Maintains same frame size for compatibility

- Supports both half-duplex (CSMA/CD) and full-duplex modes

Features:

| Feature | Description |
| --- | --- |
| Data Rate | 100 Mbps |
| Topology | Star |
| Medium | UTP or fiber optic |
| Access Method | CSMA/CD (only in half-duplex) |
| Frame Format | Same as 10 Mbps Ethernet |

Common Variants:

| Standard | Medium | Max Length | Notes |
| --- | --- | --- | --- |
| 100BASE-TX | Cat 5 UTP | 100 m | Most commonly deployed |
| 100BASE-FX | Fiber optic | 412 m | Multimode fiber |
| 100BASE-T4 | Cat 3 UTP (4 pairs) | 100 m | Used older wiring, now obsolete |

## 3. IEEE 802.3z / 802.3ab – 1 Gbps Ethernet (Gigabit Ethernet)

Overview:

- Provides 1 Gbps bandwidth

- Uses existing twisted pair or fiber

- Suitable for backbones and high-performance LANs

Features:

| Feature | Description |
| --- | --- |
| Data Rate | 1000 Mbps (1 Gbps) |
| Topology | Star |
| Medium | UTP (Cat 5e/6), fiber optic |
| Access Method | CSMA/CD (half-duplex), or full-duplex |
| Frame Format | Same as standard Ethernet |

Common Variants:

| Standard | Medium | Max Length | Notes |
| --- | --- | --- | --- |
| 1000BASE-SX | Multimode fiber | 220–550 m | Short wavelength laser |
| 1000BASE-LX | Single-mode fiber | Up to 5 km | Long wavelength laser |
| 1000BASE-T | Cat 5e/6 UTP | 100 m | Common in enterprise LANs |

## 4. IEEE 802.3ae – 10-Gbps Ethernet (10-Gigabit Ethernet)

Overview:

- Operates at 10 Gbps

- Full-duplex only; no CSMA/CD

- Suitable for backbone and data center networks

Features:

| Feature | Description |
| --- | --- |
| Data Rate | 10 Gbps |
| Topology | Point-to-point (star-based) |
| Medium | Primarily fiber; copper for short links |
| Access Method | Full-duplex only, no CSMA/CD |
| Frame Format | Same as earlier Ethernet |

Common Variants:

| Standard | Medium | Max Length | Notes |
| --- | --- | --- | --- |
| 10GBASE-SR | Multimode fiber | 26–300 m | Short range |
| 10GBASE-LR | Single-mode fiber | 10 km | Long range |
| 10GBASE-ER | Single-mode fiber | 40 km | Extended range |
| 10GBASE-T | Cat 6a/7 UTP | 55–100 m | Copper-based, backward compatible |

## Summary Table – Ethernet Evolution

| Feature | 10 Mbps | 100 Mbps | 1 Gbps | 10 Gbps |
| --- | --- | --- | --- | --- |
| IEEE Standard | 802.3 | 802.3u | 802.3z / ab | 802.3ae |
| Nickname | Ethernet | Fast Ethernet | Gigabit Ethernet | 10G Ethernet |
| Media | Coax/UTP/Fiber | UTP/Fiber | UTP/Fiber | Fiber/Copper |
| Access Method | CSMA/CD | CSMA/CD | CSMA/CD (opt) | Full-duplex only |
| Topology | Bus/Star | Star | Star | Star/PtP |
| Max Length | Up to 2 km | Up to 412 m | Up to 5 km | Up to 40 km |