# CSMA/CD

## CSMA/CD – Summary

CSMA/CD improves upon CSMA by reducing the time wasted during collisions. In basic CSMA, when two stations transmit simultaneously, their frames collide, and the entire transmission time is wasted. CSMA/CD minimizes this waste by detecting collisions early and aborting the transmission.

### CSMA/CD Procedure:

Listen: If the medium is idle, transmit; else, wait.

Wait: If the medium is busy, wait until it is idle, then transmit immediately.

Collision Detection: If a collision occurs during transmission, send a short jamming signal to notify all stations and stop transmitting.

Backoff and Retry: Wait for a random backoff time (using binary exponential backoff) before trying again.

### Collision Detection Time:

The maximum time to detect a collision is twice the end-to-end propagation delay between the two farthest stations.

### Minimum Frame Size:

Frames must be long enough to allow collision detection before the transmission ends. Otherwise, collisions may go undetected, reducing efficiency.

### Persistence Strategy:

IEEE 802.3 uses the 1-persistent strategy: transmit immediately when the channel is sensed idle.

This could cause instability (due to all stations trying to transmit immediately), but:

Collisions are short-lived if frames are long.

Random backoff helps reduce repeated collisions.

### Binary Exponential Backoff:

After each collision, the station waits a random time.

The mean of this random backoff doubles after each of the first 10 collisions.

It remains fixed for the next 6.

After 16 failed attempts, the station gives up and reports an error.

### Performance Insights:

The 1-persistent strategy with binary exponential backoff offers:

High efficiency under low load (quick access to medium).

Stability under high load (through increasing backoff).

However, it can cause a last-in-first-out effect, where recently collided stations get quicker access than those that have waited longer.

- Why Must Frames Be Long Enough in CSMA/CD?

In CSMA/CD, a station continues to monitor the medium while it is transmitting. If a collision happens, the station detects the collision and stops transmitting to avoid wasting time.

#### Problem If Frame Is Too Short:

If the frame is too short, the transmission might finish before a collision signal from another station has time to propagate back to the sender. In this case:

- The station thinks the transmission was successful.

- Collision goes undetected.

- The frame is lost, but no retransmission occurs.

- This lowers efficiency and can lead to data loss.

#### Example (Maximum Collision Detection Time):

- Suppose two stations are at opposite ends of a network.

- One station starts transmitting, and just before the signal reaches the second station, the second station starts transmitting too.

- A collision occurs, but it takes time (up to 1 propagation delay) for the signal to reach the second station, and another delay to travel back to the first.

- Hence, collision detection time = 2 × propagation delay.

Solution: Minimum Frame Size

To ensure that collisions are always detected, the frame must be long enough to keep the station transmitting for at least this round-trip time (2 × propagation delay).

This gives time for:

- The signal to travel to the farthest station.

- Any collision to occur and propagate back.

- The sender to detect the collision while still transmitting.

### Conclusion:

The minimum frame size in Ethernet (64 bytes) is specifically chosen to ensure that the station is still transmitting when a possible collision is detected from the farthest point on the network. Without this, CSMA/CD becomes unreliable, making the protocol no better than basic CSMA.