# Token Ring (IEEE 802.5)

# Introduction

Token Ring is a LAN protocol developed by IBM and standardized as IEEE 802.5. It uses a ring topology and a token-passing mechanism for medium access. Data travels in one direction around the ring from one device to the next.

# Key Features

| Feature | Token Ring |
| --- | --- |
| Topology | Logical Ring (usually physically a star) |
| Access Method | Token Passing |
| Standard | IEEE 802.5 |
| Speed | Typically 4 Mbps or 16 Mbps |
| Medium | Twisted pair or shielded coaxial cable |
| Max. Stations | 260 stations (practical limit) |

# Working Principle

A special frame called a token circulates continuously in the ring. Only the station holding the token is allowed to transmit. After transmission, the station releases the token so other stations can use the network.

Steps:

Idle Ring: Token circulates when no station is transmitting.

Frame Transmission: Station waits for the token, captures it and sends a data frame.

Frame Circulation: Frame is checked by each station; destination copies and acknowledges it.

Token Regeneration: The sender removes the frame and releases a new token.

# Token Frame Format

| Field | Description |
| --- | --- |
| Start Delimiter | Marks start of token |
| Access Control | Priority and reservation bits |
| Frame Control | Indicates token or data |
| Destination Address | MAC address of receiver |
| Source Address | MAC address of sender |
| Data | Payload |
| FCS | Frame Check Sequence (error check) |
| End Delimiter | Marks end of token/data frame |

# Advantages

Collision-Free: Only one station transmits at a time.

Deterministic Access: Guaranteed access without contention.

Fairness: All stations get equal opportunity.

High Reliability: Easier to detect and isolate faults.

# Disadvantages

Latency: Wait time for token can increase under load.

Complex Management: Token maintenance adds overhead.

Single Point of Failure: Ring break can halt the network (unless using active hubs).

# Comparison with Ethernet (CSMA/CD)

| Feature | Token Ring | Ethernet (CSMA/CD) |
| --- | --- | --- |
| Access Method | Token Passing | Carrier Sense with Collision Detection |
| Collision | No | Yes |
| Determinism | Yes | No |
| Efficiency | Better under high load | Better under low load |
| Cost | Higher | Lower |

# Priority System

IEEE 802.5 supports 8 priority levels. Stations can reserve a higher priority for urgent messages. This ensures Quality of Service (QoS) in certain applications.

# Physical Implementation

Often implemented using star-wired ring topology. All stations connect to a central hub called MAU (Multistation Access Unit). The MAU maintains the logical ring even if one device fails (bypass feature).

# Conclusion

Token Ring provides orderly and fair medium access using a token-passing protocol, suitable for networks requiring predictable performance. However, due to cost and complexity, it has largely been replaced by modern Ethernet technologies.