# Local Area Network (LAN) Overview

# Introduction to LANs

LANs (Local Area Networks) are privately owned networks used to interconnect computers and devices within a limited area such as a building or campus. In contrast to Wide Area Networks (WANs), which can be public or private, LANs are usually owned and operated by a single organization.
LANs typically have:
- Greater data transmission capacity than WANs
- Lower communication costs per unit of data
- Higher internal communication loads due to file sharing, printing, client-server computing, etc.

# 15.1 Background and Applications of LANs

## 1. Personal Computer LANs

Widely used in organizations of all sizes to support departmental applications. These LANs allow sharing of files, printers, and efficient communication. They support connection to central facilities and enable client-server applications.
Key points:
- Efficient sharing of hardware and software
- Support for centralized and peer-to-peer access
- Low cost of network attachment is crucial for widespread use

## 2. Backend Networks and Storage Area Networks (SANs)

a. Backend Networks

- Used to connect high-end systems like mainframes and storage
- High data rate (≥ 100 Mbps) and low latency required
- Operate over short distances with limited number of devices
- Support load balancing and fault tolerance

b. Storage Area Networks (SANs)

- Dedicated high-speed networks to connect servers and storage
- SAN detaches storage from specific servers
- Improves client-to-storage access and backup efficiency
- Commonly uses Fibre Channel

## 3. High-Speed Office Networks

Modern office applications (e.g., image processing, graphics) require high-speed LANs. Older LANs (e.g., 10 Mbps Ethernet) are insufficient.
- High-speed LANs are needed to handle gigabyte-level desktop storage and multimedia traffic.

## 4. Backbone LANs

Designed to interconnect multiple LANs across a building or campus.
Drawbacks of a single LAN:
- Reliability issues
- Limited capacity
- Cost inefficiencies
Backbone LANs provide high-capacity interconnection for department-level LANs.

## Summary of LAN Application Types and Key Requirements

| LAN Type | Purpose | Key Requirements |
| --- | --- | --- |
| Personal Computer LAN | Departmental computing, file and printer sharing | Low cost of attachment, support for client/server |
| Backend Network | Connect mainframes and storage for bulk data | High speed, low latency, limited devices |
| Storage Area Network (SAN) | Dedicated high-speed storage connectivity | Direct access, efficient backup, scalable |
| High-Speed Office Network | Modern multimedia office environments | Very high data rate, broad coverage |
| Backbone LAN | Interconnect multiple LANs within an organization | High capacity, scalability, fault isolation |