# Local Area Networks (LANs) – Key Concepts and Architecture

# 1. Basic Definition of LAN

A Local Area Network (LAN) is a network that connects computers and other devices within a limited geographical area, such as a home, school, office building, or campus.
It consists of:
- A shared transmission medium (e.g., copper wire, fiber optics, or wireless)
- Hardware and software components that interface devices to the medium
- Mechanisms for regulating orderly access to the shared medium

# 2. LAN Topologies

LANs can be arranged in various physical topologies including Ring, Bus, Tree, and Star.

a. Ring Topology

- Devices or repeaters are connected in a closed-loop
- Data circulates in one or both directions
- Frames travel around the ring to be picked up by destination or removed by source

b. Bus Topology

- All devices share a single central cable
- Passive medium; all stations hear all transmissions
- Suitable for small networks

c. Tree Topology

- A variation of the bus topology in a hierarchical structure
- Passive medium with branching nodes

d. Star Topology

- All devices connect to a central node (hub or switch)
- Central node controls traffic
- Failure in one link does not affect the rest of the network

# 3. LAN Standards

LAN standards define specifications for:
- Data rates (e.g., 10 Mbps, 100 Mbps, 1 Gbps, 10 Gbps)
- Topologies (e.g., star, bus, ring)
- Transmission media (e.g., twisted pair, coaxial cable, fiber optic, wireless)
Common standards include IEEE 802.3 (Ethernet) and IEEE 802.11 (Wi-Fi).

# 4. Interconnecting LANs

Organizations often operate multiple LANs that must be interconnected. A bridge is a simple device to connect two LANs:
- Operates at the data link layer
- Filters and forwards traffic using MAC addresses
- Helps segment traffic to reduce congestion

# 5. LAN Devices: Hubs and Switches

a. Hubs

- Simple device to connect multiple devices
- Operates at the physical layer
- Broadcasts data to all ports

b. Switches

- More advanced device operating at the data link layer
- Learns MAC addresses and forwards data to the correct port
- Improves efficiency and reduces unnecessary traffic

# 6. Summary Table: LAN Components & Characteristics

| Feature | Description |
| --- | --- |
| Shared Medium | Transmission medium common to all connected devices |
| Topologies | Ring, Bus, Tree, Star |
| Data Transmission | Usually in frames, with protocols for access control |
| Standards | Define data rates, topologies, and media (e.g., IEEE 802 series) |
| Interconnection | Bridges used to link LANs within an organization |
| Core Devices | Hubs (basic) and Switches (intelligent, efficient forwarding) |