# Types of Connections and Network Topologies

# Type of Connection

In a network, devices (also called nodes) need a way to be connected to facilitate communication. The way these devices are connected is referred to as the type of connection.

Two fundamental types of connections:
- Point-to-Point
- Multipoint

# Point-to-Point Connection

Definition: A point-to-point connection provides a dedicated link between two devices.

Characteristics:
- Most straightforward connection type.
- Entire capacity of the channel is reserved.
- Can be wired or wireless.

Example:
- USB connection between computer and printer.
- Telephone call between two people.

# Multipoint Connection

Definition: In a multipoint (multidrop) connection, more than two devices share a single link.

Characteristics:
- Channel is shared among multiple devices.
- Only one device can send data at a time.
- Can be spatially shared or time-shared.

Example:
- Shared projector among multiple computers.

# Physical Topology

Definition: Physical topology refers to the geometric arrangement of devices and cables in a network.

Common topologies:
1. Mesh
2. Star
3. Bus
4. Ring

# Mesh Topology

Definition: Every device is connected to every other device with a dedicated point-to-point link.

Links required: For n devices: n(n – 1)/2; each device needs n – 1 ports.

Advantages:
- Robust
- Secure
- Fault identification is easy.

Disadvantages:
- High cost
- Difficult installation and maintenance.

Used in:
- Military and financial systems.

# Star Topology

Definition: Each device has a dedicated point-to-point link to a central controller (hub or switch).

Advantages:
- Easy to install and manage
- Fault isolation is simple
- Easy to add/remove devices.

Disadvantages:
- Central node is a single point of failure.
- Requires more cabling than bus.

Used in:
- Modern Ethernet LANs.

# Bus Topology

Definition: All devices are connected to a single communication line (the bus) using drop lines and taps.

Advantages:
- Easy installation
- Less cabling.

Disadvantages:
- Difficult fault diagnosis
- A single fault can crash the entire network.

Used in:
- Legacy Ethernet networks.

# Ring Topology

Definition: Each device is connected to two other devices, forming a circular pathway.

Advantages:
- Predictable performance
- Prevents collisions using token.

Disadvantages:
- A break disrupts the entire network
- Adding/removing devices affects the ring.

Used in:
- FDDI, Token Ring LANs.

# Summary Table

| Topology | Cable Required | Installation Cost | Fault Isolation | Robustness |
| --- | --- | --- | --- | --- |
| Mesh | Very High | Very High | Excellent | Very High |
| Star | Moderate | Easy | Moderate | Moderate |
| Bus | Low | Low | Difficult | Low |
| Ring | Moderate | Moderate | Difficult | Moderate |