# Network Types and Switching

# 1. Network Types

Networks can be classified based on their geographical span and purpose. The major types include:

## 1.1 Local Area Network (LAN)

A LAN is a network that connects computers within a limited area such as a home, school, or office building.

Characteristics:
- High data transfer rates (typically 100 Mbps to 10 Gbps).
- Covers a small geographical area.
- Usually privately owned and managed.
- Uses Ethernet, Wi-Fi as communication technologies.
- Common LAN topologies include bus, star, and ring.

Example: Office network connecting desktops, printers, and servers.

## 1.2 Metropolitan Area Network (MAN)

A MAN covers a larger area than a LAN, such as a city or a campus.

Characteristics:
- Spans city-wide or campus-wide networks.
- Can be public or private.
- Acts as a high-speed network to allow sharing of regional resources.

Example: A cable TV network in a city.

## 1.3 Wide Area Network (WAN)

A WAN spans a large geographical area, often a country or continent.

Characteristics:
- Lower data transfer rates compared to LANs.
- Utilizes routers and public telecommunication links.
- Interconnects LANs and MANs.
- Can be private (enterprise WANs) or public (the Internet).

Example: The Internet.

## 1.4 Comparison between LAN and WAN

| Feature | LAN | WAN |
| --- | --- | --- |
| Area Covered | Small (building, campus) | Large (cities, countries) |
| Ownership | Usually one entity | Multiple entities |
| Data Rate | High (up to 10 Gbps) | Lower (Kbps to Gbps) |
| Delay | Low | Higher |
| Technologies | Ethernet, Wi-Fi | Frame Relay, ATM, MPLS |

# 2. Switching

Switching is a method used to forward data from source to destination in a network. The three major switching techniques are:

## 2.1 Circuit Switching

In circuit switching, a dedicated communication path is established between two parties for the duration of the session.

Phases:
1. Connection Establishment
2. Data Transfer
3. Connection Teardown

- Consistent and guaranteed transmission rate.
- Suitable for real-time voice communication.
- Inefficient for bursty data.

Example: Traditional telephone networks.

## 2.2 Packet Switching

In packet switching, data is divided into packets and transmitted independently over the network.

Types:
- Datagram Packet Switching: Packets routed independently (e.g., Internet).
- Virtual Circuit Packet Switching:

All packets follow the same logical path.
- Efficient and scalable.
- Suitable for data networks.

2.3 Circuit-Switched vs Packet-Switched Networks

| Feature | Circuit Switching | Packet Switching |
| --- | --- | --- |
| Path | Fixed path established | Dynamic routing |
| Resource Allocation | Dedicated | On demand |
| Setup Time | Required | Not required (for datagram) |
| Delay | Constant | Variable |
| Efficiency | Low | High |
| Complexity | Low | High |
| Use Case | Voice communication | Data communication |