# Reference Models: OSI and TCP/IP

# Introduction

Reference models are frameworks that standardize the functions of a telecommunication or computing system. 
They help in understanding and designing how data is communicated across networks. 
Two widely recognized models are the OSI Reference Model and the TCP/IP Protocol Suite.

# 1. OSI Reference Model

The OSI (Open Systems Interconnection) model is developed by ISO to standardize networking communication. 
It is a conceptual model consisting of seven layers, each with specific roles.

OSI Layers (Top to Bottom):

7. Application – Interface to the user; provides services like email, file transfer, etc.
6. Presentation – Translates, encrypts, compresses data.
5. Session – Establishes, maintains, and terminates communication sessions.
4. Transport – Ensures reliable data transfer; error correction and flow control.
3. Network – Logical addressing and routing (e.g., IP).
2. Data Link – Frame formatting and error detection.
1. Physical – Transmission of raw bits over a medium (cables, signals).

## Encapsulation and Decapsulation

As data moves from the top layer to the bottom layer in the sender, each layer adds its own header (encapsulation). 
At the receiver, headers are removed in reverse order (decapsulation).

# 2. TCP/IP Protocol Suite

The TCP/IP model is a practical and widely used framework for real-world Internet communications. 
It consists of four layers, each responsible for specific networking tasks.

TCP/IP Layers (Top to Bottom):

4. Application – Provides services like HTTP, FTP, DNS.
3. Transport – Reliable/unreliable data transfer using TCP/UDP.
2. Internet – Logical addressing and routing (IP).
1. Network Access – Physical data transmission and framing.

## 2.1 Key Protocols in TCP/IP Suite

| Layer | Protocols |
| --- | --- |
| Application | HTTP, FTP, SMTP, DNS, DHCP |
| Transport | TCP (reliable), UDP (unreliable) |
| Internet | IP, ICMP, ARP, IGMP |
| Network Access | Ethernet, Wi-Fi, PPP |

# 3. OSI vs TCP/IP Model: Comparison

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Developed by | ISO | DARPA / DoD |
| Number of Layers | 7 | 4 |
| Protocol-Based | No (Reference model) | Yes (Protocol suite) |
| Flexibility | Rigid and theoretical | Practical and robust |
| Protocol Binding | Not tightly bound | Tightly integrated |
| Implementation | Used as a reference | Widely implemented (Internet) |

# 4. Example: Sending an Email (Using OSI Model)

- Application Layer: Email client uses SMTP.
- Presentation Layer: Formats the message.
- Session Layer: Starts and ends the session.
- Transport Layer: TCP ensures delivery.
- Network Layer: IP routes the message.
- Data Link Layer: MAC addressing and framing.
- Physical Layer: Bits sent over cable.

5. Mnemonic to Remember OSI Layers

A common mnemonic to remember OSI layers from top to bottom:

All People Seem To Need Data Processing