# TCP/IP Protocol Suite –Notes

# 1. Introduction

- TCP/IP (Transmission Control Protocol/Internet Protocol) is a hierarchical protocol suite used on the Internet.
- Composed of interactive modules, each with specific functionality.
- Originally had 4 layers (built on hardware), now generally viewed as a 5-layer model.

# 2. Layered Architecture

- Each host/device in a network is involved in different layers based on its function:
  - Hosts (e.g., computers): Use all 5 layers.
  - Routers: Use Physical, Data Link, Network layers only.
  - Switches: Use Physical and Data Link layers.

# 3. TCP/IP 5-Layer Model

| Layer | Function Scope | Data Unit |
| --- | --- | --- |
| Application | End-to-end | Message |
| Transport | End-to-end | Segment/User Datagram |
| Network | Host-to-host | Datagram |
| Data Link | Hop-to-hop | Frame |
| Physical | Hop-to-hop | Bits |

# 4. Logical Communication

- Logical connections exist between corresponding layers of source and destination.
  - Top 3 layers (App, Transport, Network): End-to-end communication.
  - Bottom 2 layers (Data Link, Physical): Hop-to-hop communication.

# 5. Layer Responsibilities

## Physical Layer

- Transfers bits across a transmission medium.
- Converts bits into signals (electrical, optical, etc.).
- Logical unit of communication: bit.

## Data-Link Layer

- Moves frames across individual links (LAN/WAN).
- Adds headers for addressing and error control.
- No specific protocol defined; supports many standards.

## Network Layer

- Responsible for host-to-host communication and routing.
- Uses IP (Internet Protocol) for addressing and delivery.
- Supports fragmentation, forwarding, and auxiliary protocols:
  - ICMP, IGMP, DHCP, ARP.

## Transport Layer

- Provides end-to-end communication for applications.
- Common protocols:
  - TCP: Reliable, connection-oriented, with flow/error/congestion control.
  - UDP: Connectionless, simple, no controls.
  - SCTP: Designed for multimedia applications.

## Application Layer

- Handles process-to-process communication.
- Protocol examples: HTTP, SMTP, FTP, DNS, TELNET, SSH, SNMP.

# 6. Encapsulation and Decapsulation

- Encapsulation: Wrapping data with headers as it moves down the stack.
- Decapsulation: Removing headers as data moves up the stack.
- End-to-End layers: Application, Transport, Network.

## Multiplexing and Demultiplexing in TCP/IP Protocol Suite

### Definition

- Multiplexing (at the source):
A lower-layer protocol can encapsulate packets from multiple upper-layer protocols, one at a time.

- Demultiplexing (at the destination):
A lower-layer protocol can decapsulate a packet and deliver it to the appropriate upper-layer protocol, based on header information.

### How It Works

- Header Field:
Each protocol doing multiplexing/demultiplexing includes a field in its header (like a port number or protocol ID) that helps identify the intended upper-layer protocol.

### Conclusion

Multiplexing and demultiplexing ensure that data from multiple applications and services can share the same network infrastructure and reach the correct destination process.