# Reference Models: OSI and TCP/IP

# Introduction

Reference models are frameworks that standardize the functions of a telecommunication or computing system. 
They help in understanding and designing how data is communicated across networks. 
Two widely recognized models are the OSI Reference Model and the TCP/IP Protocol Suite.

# 1. OSI Reference Model

The OSI (Open Systems Interconnection) model is developed by ISO to standardize networking communication. 
It is a conceptual model consisting of seven layers, each with specific roles.

OSI Layers (Top to Bottom):

7. Application – Interface to the user; provides services like email, file transfer, etc.
6. Presentation – Translates, encrypts, compresses data.
5. Session – Establishes, maintains, and terminates communication sessions.
4. Transport – Ensures reliable data transfer; error correction and flow control.
3. Network – Logical addressing and routing (e.g., IP).
2. Data Link – Frame formatting and error detection.
1. Physical – Transmission of raw bits over a medium (cables, signals).

## Encapsulation and Decapsulation

As data moves from the top layer to the bottom layer in the sender, each layer adds its own header (encapsulation). 
At the receiver, headers are removed in reverse order (Decapsulation).

### Summary: Principles of Protocol Layering

1. First Principle – Bidirectional Functionality:
Each protocol layer must handle opposite tasks in both directions to support two-way communication.

- Example:

  - Layer 3: Talk in one direction and listen in the other.

  - Layer 2: Encrypt and decrypt.

  - Layer 1: Send and receive mail.

2. Second Principle – Identical Peer Objects:
The objects handled by the same layer at both ends of communication must be identical.

- Example:

  - Layer 3 handles plaintext letters at both ends.

  - Layer 2 handles cipher text letters.

  - Layer 1 handles physical mail.

### Logical Connections:

Following these two principles creates the idea of logical (imaginary) connections between corresponding layers at the sender and receiver ends.
These logical connections help us understand layer-to-layer communication, a key concept in data communication and networking.

# OSI vs TCP/IP Model: Comparison

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Developed by | ISO | DARPA / DoD |
| Number of Layers | 7 | 4 |
| Protocol-Based | No (Reference model) | Yes (Protocol suite) |
| Flexibility | Rigid and theoretical | Practical and robust |
| Protocol Binding | Not tightly bound | Tightly integrated |
| Implementation | Used as a reference | Widely implemented (Internet) |

# 4. Example: Sending an Email (Using OSI Model)

- Application Layer: Email client uses SMTP.
- Presentation Layer: Formats the message.
- Session Layer: Starts and ends the session.
- Transport Layer: TCP ensures delivery.
- Network Layer: IP routes the message.
- Data Link Layer: MAC addressing and framing.
- Physical Layer: Bits sent over cable.

5. Mnemonic to Remember OSI Layers

A common mnemonic to remember OSI layers from top to bottom:

All People Seem To Need Data Processing