Network links are of two types: point-to-point connections and broadcast channels. While point-to-point links were studied earlier, broadcast links require special protocols to manage channel sharing among multiple users.

The central issue in broadcast networks is channel access control—deciding who gets to transmit when multiple users compete for the medium. This problem is similar to several people talking simultaneously in a conference call, leading to chaos if not managed.

Protocols that resolve this issue form the Medium Access Control (MAC) sublayer of the data link layer. The MAC sublayer is especially significant in Local Area Networks (LANs), particularly wireless LANs, since wireless communication is naturally broadcast. Wide Area Networks (WANs) generally rely on point-to-point links, except in the case of satellite networks which use broadcast.

Although the MAC sublayer lies at the lower end of the data link layer, it is discussed after point-to-point protocols for better understanding, since multi-party communication builds on the principles of two-party communication.

## The Channel Allocation Problem

The main challenge in broadcast networks is how to share a single communication channel among multiple competing users. Whether the medium is a segment of wireless spectrum or a shared wire/fiber, any user’s full use of the channel prevents others from accessing it.

### 1. Static Channel Allocation

- Traditional Method: Uses multiplexing schemes like FDM (Frequency Division Multiplexing) or TDM (Time Division Multiplexing) to divide the channel into fixed portions. Each user gets a dedicated subchannel, avoiding interference.

- Efficient only when:

  - The number of users is small and constant.

  - Traffic is steady and predictable (e.g., FM radio stations).

- Problems with bursty traffic:

  - Inefficiency: Idle users waste their allocated bandwidth.

  - Blocking: New users may be denied access if all slots are assigned, even if some are underutilized.

  - High delays: Queueing analysis shows that dividing capacity into N subchannels increases mean delay by a factor of N.

- Conclusion: Static allocation is poorly suited for computer networks, where traffic is bursty with large peak-to-average ratios.

### 2. Assumptions for Dynamic Channel Allocation

Dynamic schemes aim to adaptively share the channel. Their design typically relies on the following five assumptions:

- Independent Traffic: Each of the N stations generates frames randomly (Poisson arrivals). A station waits until its frame is successfully transmitted.

- Single Channel: All stations use the same channel for transmitting and receiving. No external coordination exists.

- Observable Collisions: If two transmissions overlap, a collision occurs. Collided frames are destroyed and must be retransmitted. (Collision detection is easier in wired than wireless systems.)

- Continuous or Slotted Time:

  - Continuous: Frames can be sent anytime.

  - Slotted: Time is divided into slots; transmissions start only at slot boundaries. Slotted systems reduce waste but require synchronization.

- Carrier Sense or No Carrier Sense:

  - With carrier sense: Stations detect whether the channel is busy before transmitting.

  - Without carrier sense: Stations transmit blindly and later detect success or collision.

### Key Insight

- Static methods fail under bursty traffic.

- Dynamic allocation protocols are essential to efficiently manage shared broadcast channels, balancing fairness, throughput, and delay.