# Carrier Sense Multiple Access (CSMA) Protocols

# 1. Motivation

In ALOHA systems, stations transmit without knowing what others are doing, leading to many collisions. The best channel utilization in slotted ALOHA is 1/e ≈ 0.37.

Improvement idea:
 - In LANs, stations can listen to the channel before transmitting.
 - If another transmission is in progress, a station can wait, reducing collisions.

Such protocols are called Carrier Sense Multiple Access (CSMA) protocols.

# 2. Basic Principle of CSMA

- Carrier Sense: Each station listens to the channel before transmitting.
 - Multiple Access: Many stations share the same channel.
 - If channel is idle → transmission proceeds.
 - If channel is busy → station waits or backs off.
Collisions can still occur because of propagation delay.

# 3. Types of CSMA Protocols

## 3.1 1-Persistent CSMA

Algorithm:
 1. If the channel is idle, the station transmits immediately (probability = 1).
 2. If the channel is busy, the station waits until it becomes idle and then transmits.
 3. On collision, the station waits for a random backoff and tries again.

Features:
 - Greedy approach.
 - Collisions occur when multiple stations wait and then transmit simultaneously, or due to propagation delay.
 - Better performance than ALOHA, but worse in channels with high bandwidth-delay product.

## 3.2 Non-Persistent CSMA

Algorithm:
 1. If the channel is idle, transmit immediately.
 2. If the channel is busy, wait for a random backoff time before sensing again.

Features:
 - Less greedy than 1-persistent.
 - Reduces simultaneous collisions.
 - Higher average delay but better channel utilization.

## 3.3 p-Persistent CSMA

Algorithm (slotted channels):
 1. Sense the channel.
 2. If idle:
    - Transmit with probability p.
    - Defer to next slot with probability q = 1 − p.
 3. If busy, wait until next slot and repeat.
 4. On collision, wait random time and restart.

Features:
 - Balances greediness and patience.
 - High p → behaves like 1-persistent.
 - Low p → fewer collisions but higher delay.
 - Used in IEEE 802.11 (Wi-Fi) with refinements like exponential backoff.

# 4. Comparison of CSMA Protocols

| Protocol | Behavior when idle | Behavior when busy | Pros | Cons |
| --- | --- | --- | --- | --- |
| 1-Persistent | Transmit immediately | Wait until free, then transmit | Simple, better than ALOHA | High collisions due to greediness |
| Non-Persistent | Transmit immediately | Wait random time before retry | Fewer collisions | Higher average delay |
| p-Persistent | Transmit with probability p | Repeat in next slot | Balances delay & collisions | Needs slotted channel, tuning of p |

# 5. Effect of Propagation Delay

Collisions are not fully eliminated due to propagation delay. Example: Station A begins sending, Station B senses before the signal arrives and transmits, leading to collision.

Severity depends on bandwidth-delay product:
 - Small: fewer collisions (LANs).
 - Large: more collisions (long-distance networks).

# 6. Throughput Performance

Throughput order (best to worst):
 Non-persistent CSMA > p-persistent CSMA > 1-persistent CSMA > Slotted ALOHA > Pure ALOHA.

CSMA protocols achieve much higher channel utilization than ALOHA.

# Summary

CSMA protocols improve upon ALOHA by sensing the channel before transmitting.
 - 1-persistent: greedy but simple.
 - Non-persistent: fewer collisions but more delay.
 - p-persistent: balances delay and collisions, widely used (e.g., Wi-Fi).

Propagation delay limits collision avoidance, but CSMA achieves much better utilization than ALOHA.