# Carrier Sense Multiple Access with Collision Detection (CSMA/CD)

# 1. Motivation

ALOHA protocols had poor channel utilization:
 - Pure ALOHA max throughput ≈ 18%.
 - Slotted ALOHA max throughput ≈ 37%.

Persistent and non-persistent CSMA improved utilization by listening before transmitting. However, if two stations sense the channel to be idle and transmit simultaneously, collisions still occur.

Idea: Do not finish transmitting once a collision occurs. Abort transmission immediately after detecting a collision. This strategy saves time and bandwidth.

This technique is Carrier Sense Multiple Access with Collision Detection (CSMA/CD), the foundation of classic Ethernet (IEEE 802.3).

# 2. Collision Detection: How It Works

- Analog process: Station listens to the channel while transmitting.
 - If transmitted signal ≠ received signal → collision detected.

Requirements:
 - Received signal must not be extremely weaker than transmitted signal (problem in wireless).
 - Modulation scheme must allow collisions to be detectable (e.g., voltages must show interference).

Example: A 0V + 0V collision may be undetectable.

# 3. Conceptual Model (Fig. 4-5)

CSMA/CD operates in 3 states:
 1. Contention Period – multiple stations attempt transmission, collisions possible.
 2. Transmission Period – one station seizes channel and transmits frame fully.
 3. Idle Period – no transmission (stations have no data).

# 4. Contention and Collision Timing

Key parameter: Propagation time (τ) between farthest stations.

Worst-case collision detection:
 - At t0: Station A begins transmitting.
 - At t0 + τ − ε: Station B starts (just before A’s signal arrives).
 - B detects collision immediately and stops, but noise reaches A only at t0 + 2τ − ε.

Conclusion:
 - A station must transmit for at least 2τ without hearing a collision before it has seized the channel.
 - Thus, slot time = 2τ in CSMA/CD.

# 5. Relation to Slotted ALOHA

CSMA/CD contention resembles slotted ALOHA, but slot = 2τ.

Difference:
 - In CSMA/CD, once a station seizes the channel, it transmits the full frame.
 - In ALOHA, transmission is restricted to slots.

Result: Much higher throughput, especially when frame length ≫ propagation time.

# 6. Random Access Protocol Comparisons (Fig. 4-4)

| Protocol | Max Throughput | Key Characteristics |
| --- | --- | --- |
| Pure ALOHA | ≈ 18% | Random send, no slots, high collision rate |
| Slotted ALOHA | ≈ 37% | Synchronization in slots, fewer collisions |
| 1-Persistent CSMA | Higher than ALOHA | Always transmits when idle, collision-prone |
| Non-Persistent CSMA | Better than 1-persistent | Waits random time, less greedy |
| p-Persistent CSMA | Adjustable | Transmits with probability p when idle |
| CSMA/CD | Best among these | Detects collisions early, aborts transmission (Ethernet) |

Example p values:
 - 0.01-persistent CSMA → very patient (low collisions, high delay).
 - 0.1-persistent CSMA → moderate.
 - 0.5-persistent CSMA → balanced.
 - 1-persistent CSMA → greedy (more collisions).

# 7. Advantages of CSMA/CD

- Minimizes wasted channel capacity by stopping collided transmissions quickly.
 - Provides efficient utilization for wired LANs.
 - Foundation of Ethernet’s Binary Exponential Backoff algorithm.

# 8. Limitations of CSMA/CD

- Ineffective in wireless networks due to weak received signals.
 - Requires shared medium (coaxial cable, bus topology).
 - Largely obsolete today due to switched Ethernet.

# Summary

CSMA/CD improves CSMA by detecting and aborting collisions early. With a slot size of 2τ, it prevents long wasted transmissions, leading to higher throughput than ALOHA and CSMA variants. It was central to Ethernet’s success in LANs, though modern switched Ethernet no longer requires it.