## Assignment – 4

### UNIT: Network Layer

1. Explain the main functions of the Network Layer. Describe how routers perform packet forwarding and routing in an internetwork.

2. Differentiate between the Network Layer and Data Link Layer services. Explain why the network layer is responsible for host-to-host delivery.

3. Define routing. Explain the shortest-path routing and flooding techniques used in computer networks with suitable examples.

4. Explain the principles of distance vector and link state routing algorithms (no algorithmic steps needed). Compare their advantages and limitations.

5. Discuss the causes of network congestion. Explain the approaches to congestion control in the network layer.

6. Explain the concept of Quality of Service (QoS) in computer networks. Describe the parameters used to measure QoS and their significance.

7. Compare IPv4 and IPv6 addressing. Discuss their header formats, address lengths, and major improvements introduced in IPv6.

### UNIT: Transport Layer

- 8. Explain the services provided by the Transport Layer. Compare TCP and UDP in terms of connection, reliability, flow control, and use cases.

- 9. Explain the elements of transport protocols. Discuss the role of flow control and error control in reliable data transmission.

- 10. Explain the concept of connection establishment and termination in TCP using the three-way and four-way handshakes.

Answer 6 questions in total, with minimum two from Transport layer.

Main Functions of the Network Layer

The Network Layer (Layer 3 of the OSI model) is responsible for the delivery of packets from the source host to the destination host across multiple interconnected networks (an internetwork). It focuses on logical addressing and path determination.

1. Logical Addressing

- Each device on a network is assigned a unique IP address (logical address).

- The network layer uses these addresses to identify the source and destination of each packet002E

- Example: In the Internet, the IP protocol provides these logical addresses (IPv4 or IPv6).

2. Routing

- Routing is the process of determining the best path for data to travel from the source to the destination.

- The network layer uses routing algorithms (like Dijkstra or Bellman-Ford) and routing protocols (like OSPF, RIP, BGP) to make these decisions.

- It considers metrics such as hop count, bandwidth, delay, and reliability.

3. Packet Forwarding

- After determining the route, the network layer forwards packets from one network to another toward their destination.

- Each router examines the packet’s destination IP address and forwards it to the next appropriate router (next hop).

4. Fragmentation and Reassembly

- Different networks may have different Maximum Transmission Units (MTU).

- The network layer may fragment large packets into smaller ones to fit the MTU.

- The destination host’s network layer reassembles these fragments into the original packet.

5. Error Handling and Diagnostics

- Provides mechanisms for error reporting and diagnostics.

- Example: The ICMP (Internet Control Message Protocol) is used to send error messages (e.g., “Destination Unreachable”, “Time Exceeded”).

How Routers Perform Packet Forwarding and Routing

Routers are the primary devices that operate at the network layer. Their two main tasks are routing and forwarding.

1. Routing (Path Determination)

- Routing Table Creation:
Routers maintain a routing table containing entries for destination networks and the best next hop to reach them.

- Routing Protocols:
Routers exchange information with other routers using routing protocols (e.g., OSPF, RIP, BGP) to learn about network topology.

- Dynamic Updates:
The routing table is dynamically updated when network conditions change (e.g., link failure).

2. Packet Forwarding

Once the route is known:

- The router receives a packet on one of its interfaces.

- It examines the destination IP address in the packet header.

- It looks up the destination network in its routing table.

- It determines the next hop (the next router or final destination).

- It encapsulates the packet into a new frame and sends it out through the appropriate interface.

Example

Suppose Host A (192.168.1.10) sends data to Host B (10.0.0.5):

- The packet leaves Host A and reaches Router R1.

- R1 checks its routing table and sees that packets for network 10.0.0.0/8 should go to Router R2.

- R1 forwards the packet to R2.

- R2 repeats the same process until the packet reaches the network where Host B resides.

Summary Table

| Function | Description |
| --- | --- |
| Logical Addressing | Assigns IP addresses to identify source and destination |
| Routing | Determines best path to the destination |
| Packet Forwarding | Moves packets from source to destination via routers |
| Fragmentation/Reassembly | Adjusts packet size to fit MTU limits |
| Error Handling | Uses ICMP for diagnostics and error reporting |

Routing

Routing is the process of selecting an optimal path for data packets to travel from a source to a destination through an internetwork.
It determines which route packets should take based on factors such as hop count, distance, delay, or bandwidth.
Routers perform this function by using routing algorithms and routing tables to forward packets efficiently from one network to another.

1. Shortest-Path Routing

Concept

In this technique, the network is represented as a graph where each node denotes a router and each edge represents a communication link.
Each link is assigned a cost (for example, distance or delay). The algorithm then determines the path with the minimum total cost between the source and destination.

The most common algorithm used is Dijkstra’s Algorithm, which finds the shortest path from a source node to all other nodes in the network.

Example

Consider the network below:

2

A------B

|\     |

1 | \3   |1

|  \   |

C---D--E

2

To send a packet from A to E, the possible paths and their total costs are:

- A–B–E → 2 + 1 = 3

- A–C–D–E → 1 + 2 + 2 = 5

Hence, the shortest path is A–B–E, with a total cost of 3.

Advantages

- Provides the most efficient route.

- Reduces delay and congestion.

- Guarantees optimal delivery based on defined metrics.

Disadvantages

- Requires each router to know the complete network topology.

- Updating information in large, dynamic networks can cause overhead.

2. Flooding

Concept

Flooding is a simple routing technique in which every incoming packet is sent out on all outgoing links, except the one it arrived on.
It does not require any routing tables or topology information. Every possible path is explored to ensure delivery.

Example

For a small network:

A

/ \

B---C

When node A sends a packet, it forwards copies to both B and C.
Each of them then forwards the packet to all other nodes (except the sender), ensuring that all nodes eventually receive the message.

Control Methods

To avoid infinite looping and congestion:

- Hop Counter (TTL): Each packet has a hop limit that decreases at every hop.

- Sequence Numbers: Routers keep track of received packets to discard duplicates.

- Selective Flooding: Packets are sent only in the general direction of the destination.

Advantages

- Highly reliable — if a path exists, the packet will reach the destination.

- No need for routing tables or path calculation.

- Useful for broadcasting and network discovery.

Disadvantages

- Generates duplicate packets, causing network congestion.

- Inefficient use of bandwidth and processing power.

Comparison

| Aspect | Shortest-Path Routing | Flooding |
| --- | --- | --- |
| Basis | Chooses minimum-cost path | Sends packets through all possible paths |
| Efficiency | High | Low (many duplicates) |
| Reliability | Moderate | Very high |
| Routing Table | Required | Not required |
| Use Case | Unicast communication | Broadcast or fault-tolerant networks |

Conclusion

Routing enables packets to travel efficiently across an internetwork.
Shortest-Path Routing provides optimal delivery based on path cost, while Flooding ensures guaranteed delivery by sending packets over every possible route.
Both methods form the foundation for more advanced routing protocols used in modern computer networks.

Q: Discuss the causes of network congestion. Explain the approaches to congestion control in the network layer.

Definition of Congestion

Network congestion occurs when too many packets are present in a part of the network, causing packet delay and loss, which degrade overall performance. When the offered load exceeds the network’s carrying capacity, buffers inside routers fill up, packets are dropped, and throughput decreases sharply, leading to a condition known as congestion collapse.

Causes of Network Congestion

- High Offered Load:
When hosts send data into the network at a rate higher than what it can handle, queues build up inside routers, resulting in delays and packet losses.

- Low-Bandwidth Links:
Some links may have less bandwidth than others in a communication path, forming a bottleneck that slows down packet transmission.

- Slow Routers or Processing Delays:
Routers that process packets slower than the line rate cause accumulation of packets in buffers.

- Retransmissions Due to Timeouts:
When packets are delayed, the sender may assume they are lost and retransmit them. These duplicates further overload the network.

- Insufficient Memory in Routers:
Limited buffer space leads to packet drops, while excessive buffering can worsen delays and cause congestion collapse (as noted by Nagle, 1987).

- Bursty Traffic:
Internet traffic is highly variable. Sudden bursts can overload routers even if the average load is moderate.

Approaches to Congestion Control

Tanenbaum categorizes congestion control into preventive (slower, long-term) and reactive (faster, short-term) techniques — as illustrated in Figure 5-22 of the textbook.

| Preventive (Slower) | Reactive (Faster) |
| --- | --- |
| Network Provisioning | Traffic Throttling |
| Traffic-Aware Routing | Load Shedding |
| Admission Control |  |

1. Network Provisioning

The most fundamental way to prevent congestion is to match network capacity with expected traffic. Heavily loaded links and routers are upgraded periodically. This is a long-term solution based on traffic trends.

2. Traffic-Aware Routing

Routes are adjusted based on current load conditions. By changing shortest path weights or splitting traffic over multiple paths, congestion in hotspots can be reduced.
However, rapid adjustments may lead to oscillations in routing tables.

3. Admission Control

Used mainly in virtual-circuit networks, this technique prevents new connections from being established if the network lacks sufficient resources to handle them.
Example: Telephone networks refuse new calls when overloaded.

4. Traffic Throttling

When congestion is imminent, routers send feedback signals to the sources asking them to reduce transmission rate.
Two key methods are:

- Choke Packets: Routers send a special packet to the sender instructing it to slow down.

- Explicit Congestion Notification (ECN): Instead of sending extra packets, routers mark the packet header to indicate congestion, and the receiver notifies the sender to reduce its rate.

5. Load Shedding

As a last resort, when all else fails, routers discard packets that cannot be delivered. Intelligent policies (e.g., dropping less important packets first) can prevent total congestion collapse.
A modern form of load shedding is Random Early Detection (RED), where routers drop packets early—before buffers overflow—to signal sources to slow down.

Summary

| Cause of Congestion | Control Approach |
| --- | --- |
| High load / traffic bursts | Traffic throttling, load shedding |
| Bottleneck links | Traffic-aware routing |
| Slow routers | Network provisioning |
| Too many connections | Admission control |

Conclusion

Network congestion is an inevitable challenge in data communication. By combining preventive measures (like provisioning and routing) with reactive techniques (like feedback and load shedding), networks can efficiently manage congestion and maintain optimal performance.

Q: Explain the concept of Quality of Service (QoS) in computer networks. Describe the parameters used to measure QoS and their significance.

Definition of Quality of Service (QoS)

Quality of Service (QoS) refers to the set of techniques and mechanisms that ensure a certain level of performance, reliability, and predictability for data transmission across a network.
In simple terms, QoS determines how well the network can meet the requirements of different applications, such as real-time voice, video, or data transfer.

As stated in the textbook, some applications — especially multimedia and real-time communications — require guaranteed throughput, minimal delay, and low packet loss to function effectively. QoS mechanisms help a network deliver this consistent performance even under high traffic conditions, without simply overprovisioning bandwidth.

Need for QoS

Not all network applications have the same performance needs:

- Email and file transfer can tolerate delay but need reliability.

- Voice calls and video conferencing require minimal delay and jitter.

- Streaming services need consistent throughput.

QoS ensures that each application type gets the performance it requires, maintaining user satisfaction and efficient network utilization.

Parameters Used to Measure QoS

According to Tanenbaum (Section 5.4.1 “Application Requirements”), the quality of a network connection is typically characterized by four primary parameters:

| Parameter | Meaning | Significance |
| --- | --- | --- |
| 1. Bandwidth (Throughput) | The amount of data that can be transmitted per second, measured in bits per second (bps). | Determines how much information can be sent in a given time. High bandwidth is essential for video streaming, large file transfers, etc. |
| 2. Delay (Latency) | The time taken for a packet to travel from the source to the destination. | Critical for real-time applications like voice and video calls. Excessive delay causes noticeable lag in conversations or streaming. |
| 3. Jitter (Delay Variation) | The variation in packet arrival times. | Particularly affects audio and video; irregular arrival can cause distortion or uneven playback unless buffered. |
| 4. Packet Loss (Error Rate) | The percentage of packets that fail to reach the destination. | Affects data integrity and quality. File transfers require nearly zero loss, while multimedia can tolerate small losses. |

Illustration: Application Requirements (based on Fig. 5-27 in the textbook)

| Application | Bandwidth | Delay Sensitivity | Jitter Sensitivity | Loss Tolerance |
| --- | --- | --- | --- | --- |
| Email | Low | Low | Low | Medium |
| File Sharing | High | Low | Low | Medium |
| Web Browsing | Medium | Medium | Low | Medium |
| Remote Login | Low | Medium | Medium | Medium |
| Audio on Demand | Low | Low | High | Low |
| Video on Demand | High | Low | High | Low |
| Telephony | Low | High | High | Low |
| Video Conferencing | High | High | High | Low |

(Adapted from Fig. 5-27, Tanenbaum & Wetherall, Computer Networks, 5th Edition)

Significance of QoS Parameters

- Bandwidth:
Guarantees that high-data-rate applications receive sufficient resources to operate smoothly.

- Delay:
Ensures prompt delivery of time-sensitive packets, improving interactivity and user experience.

- Jitter:
Maintains consistent packet arrival intervals, crucial for maintaining smooth playback of voice or video streams.

- Loss:
Reduces retransmissions and improves reliability for applications requiring data accuracy.

Conclusion

Quality of Service (QoS) is a crucial concept that ensures the predictable performance of networks, especially with the growing demand for real-time multimedia applications.
By managing parameters such as bandwidth, delay, jitter, and packet loss, networks can provide differentiated services to various applications — ensuring that each performs optimally according to its needs.

Explain the services provided by the Transport Layer. Compare TCP and UDP in terms of connection, reliability, flow control, and use cases.

Transport Layer Overview

The Transport Layer acts as a bridge between the network layer and the application layer. Its main function is to provide end-to-end communication between processes running on different hosts. It ensures that data is delivered correctly, in order, and at an acceptable rate to the intended process, regardless of the underlying physical network (Tanenbaum, Computer Networks, 5th Ed., Sec. 6.1.1).

Services Provided by the Transport Layer

According to Tanenbaum, the ultimate goal of the transport layer is to provide efficient, reliable, and cost-effective data transmission service to its users (typically application processes). The key services include:

- Process-to-Process Communication:
While the network layer delivers packets host-to-host, the transport layer delivers data process-to-process, using port numbers to identify applications (e.g., HTTP uses port 80).

- Connection Establishment and Release:
The transport layer sets up logical connections before data transfer and releases them gracefully afterward.
(Reference: Section 6.1.2 and Figure 6-2, “Transport Service Primitives.”)

- Reliable Data Transfer:
Provides error detection, acknowledgment, and retransmission mechanisms to ensure that data is delivered without corruption or loss (Section 6.2.4).

- Flow Control:
Prevents a fast sender from overwhelming a slow receiver by regulating the data flow between them (explained using the sliding window technique in Section 6.5.8).

- Multiplexing and Demultiplexing:
Allows multiple applications to share the same network connection simultaneously. Each message includes a source and destination port to ensure proper delivery to the right application.

- Error Control:
Uses checksums, acknowledgments (ACKs), and retransmissions to detect and correct errors during transmission (Section 6.2.4).

- Congestion Control:
Regulates data flow to prevent excessive load in the network, especially in TCP connections (Section 6.3).

Transport Layer Protocols

The Internet provides two main transport protocols:

- Transmission Control Protocol (TCP) — Connection-oriented and reliable.

- User Datagram Protocol (UDP) — Connectionless and unreliable.

(Referenced from Section 6.4: “The Internet Transport Protocols: UDP” and Section 6.5: “The Internet Transport Protocols: TCP.”)

Comparison of TCP and UDP

| Feature | TCP (Transmission Control Protocol) | UDP (User Datagram Protocol) |
| --- | --- | --- |
| Connection Type | Connection-oriented — a virtual connection is established using a three-way handshake before data transfer. | Connectionless — packets are sent independently without prior setup. |
| Reliability | Reliable — provides acknowledgment, sequencing, retransmission, and error recovery. | Unreliable — no acknowledgment, sequencing, or retransmission. If a packet is lost, it is not recovered. |
| Flow Control | Implements sliding window mechanism for flow control (variable window size). Prevents receiver overflow. | No flow control; sender transmits at its own rate. |
| Error Control | Uses checksum for error detection and retransmission for correction. | Uses optional checksum only for error detection (no retransmission). |
| Ordering of Data | Ensures packets are delivered in correct sequence. | No guarantee of packet order. |
| Congestion Control | Implements congestion control algorithms like slow start and congestion avoidance (Section 6.5.10). | No congestion control; application must manage it if required. |
| Header Size | 20 bytes (minimum) — more overhead due to control fields. | 8 bytes — simpler and faster. |
| Use Cases | Suitable for applications requiring reliability, e.g., web browsing (HTTP/HTTPS), email (SMTP, IMAP), file transfer (FTP). | Suitable for applications needing speed over reliability, e.g., video streaming, VoIP, online gaming, DNS queries. |

Diagram: TCP vs UDP Relationship in Internet Model

Application Layer

↑

Transport Layer:  TCP | UDP

↑

Network Layer:    IP

↑

Data Link Layer

Figure: Relation between IP, TCP, and UDP (adapted from Fig. 1-22, Tanenbaum).

Summary

- The Transport Layer provides crucial services such as reliable delivery, flow control, multiplexing, and error recovery.

- TCP is used when accuracy and reliability are vital, ensuring error-free, ordered delivery with flow and congestion control.

- UDP is used for lightweight, time-sensitive applications where occasional loss is acceptable in exchange for lower latency.

Thus, the choice between TCP and UDP depends on the application’s need for reliability versus speed.

like the one in Fig. 6-27 and Fig. 6-33 from the book)

----------------------------------------------------------------------------------------------------------------

Q: Explain the elements of transport protocols. Discuss the role of flow control and error control in reliable data transmission.

Elements of Transport Protocols

According to Tanenbaum (Chapter 6.2, “Elements of Transport Protocols”), a transport protocol defines how two transport entities on different hosts communicate to provide reliable, efficient, and ordered data delivery between applications.
Although the transport layer shares some features with the data link layer, its operations differ because it manages end-to-end communication across a potentially large and unreliable network, rather than a single physical link.

The main elements of a transport protocol are:

- Addressing (Transport Service Access Points – TSAPs):
Identifies communicating processes using port numbers. Each application listens on a specific TSAP for connection requests (Fig. 6-8 in textbook).

- Connection Establishment:
Before transmitting data, the sender and receiver perform a handshake (e.g., TCP’s three-way handshake) to establish parameters like sequence numbers and buffer sizes.

- Data Transfer and Sequencing:
Data is transmitted in segments, each carrying sequence numbers to ensure correct ordering and detection of lost or duplicate segments.

- Flow Control:
Prevents a fast sender from overwhelming a slow receiver by regulating the amount of unacknowledged data that can be in transit.

- Error Control:
Ensures reliable data delivery through acknowledgments (ACKs), retransmissions, and checksums to detect and correct transmission errors.

- Multiplexing and Demultiplexing:
Allows multiple processes to use the same network connection by assigning different port numbers.

- Connection Release:
After communication is complete, both sides close the connection gracefully using control messages to ensure all data is delivered.

- Crash Recovery:
Mechanisms ensure proper handling of data when either side crashes during transmission, avoiding loss or duplication when the connection restarts.

(Ref: Section 6.2, pp. 508–528, Tanenbaum & Wetherall, “Computer Networks,” 5th Edition)

Role of Flow Control

As explained in Section 6.2.4 — Error Control and Flow Control,
Flow Control ensures that the sender does not send data faster than the receiver can process it.
Without it, receiver buffers could overflow, leading to data loss.

Mechanisms Used:

- Sliding Window Protocol:
Allows multiple segments to be sent before waiting for acknowledgments. The receiver advertises a “window size” that tells the sender how much data it can accept.

- Stop-and-Wait Protocol:
A simpler form where the sender transmits one segment at a time and waits for an acknowledgment before sending the next.

Significance:

- Maintains synchronization between sender and receiver speeds.

- Prevents buffer overflow at the receiver.

- Improves throughput on long, high-latency links through pipelining.

(Ref: Section 6.2.4, pp. 522–523)

Role of Error Control

Error Control ensures that data is delivered accurately and in the correct sequence.
It provides reliability even when packets are lost, duplicated, or corrupted during transmission.

Mechanisms Used:

- Checksums:
Each segment carries a checksum to detect transmission errors. If the checksum fails, the segment is discarded.

- Acknowledgments (ACKs):
The receiver confirms successful receipt of segments.

- Retransmissions (ARQ – Automatic Repeat reQuest):
If the sender does not receive an acknowledgment within a specific timeout period, it retransmits the segment.

- Sequence Numbers:
Used to identify and reorder segments and detect duplicates.

- Sliding Window for Reliability:
Integrates both flow and error control by maintaining a range of unacknowledged packets that can be retransmitted if necessary.

Significance:

- Guarantees end-to-end reliability even if intermediate routers lose or corrupt packets.

- Ensures in-order delivery of data.

- Protects against duplication or omission of packets.

(Ref: Section 6.2.4, pp. 522–524; also see discussion on ARQ and sliding window protocols.)

Illustrative Diagram (from Fig. 6-7 and Fig. 6-14 in textbook)

Sender                                  Receiver

|----[Segment 1]---->|  --> ACK 1

|----[Segment 2]---->|  --> ACK 2

|----[Segment 3]---->|  --> ACK 3

|<---Window Size=3---|

Flow and error control using sliding window mechanism in the transport layer.

Conclusion

The transport layer provides reliable, process-to-process communication by managing addressing, sequencing, flow control, and error recovery.
Flow control ensures smooth data transfer without overwhelming the receiver, while error control ensures the accuracy and reliability of the transmitted data.
Together, they form the backbone of reliable transport protocols such as TCP, ensuring dependable end-to-end delivery over an unreliable network.

Q: Explain the concept of connection establishment and termination in TCP using the three-way and four-way handshakes.

Introduction

The Transmission Control Protocol (TCP) is a connection-oriented and reliable transport protocol. Before communication begins, a connection must be established between the sender and receiver to synchronize sequence numbers and allocate resources. Once the data transfer is complete, the connection is properly terminated to ensure that all data has been delivered successfully and no packets remain in transit.

1. Connection Establishment – Three-Way Handshake

The three-way handshake is used by TCP to establish a reliable connection between two devices. It ensures both sides agree on the initial sequence numbers and are ready to send and receive data.

Steps Involved

| Step | Sender (Client) | Receiver (Server) | Description |
| --- | --- | --- | --- |
| 1. SYN | Sends a segment with SYN = 1 and an initial sequence number (Seq = x). | Waits for a connection request. | Client requests to start a connection. |
| 2. SYN + ACK | Waits for reply. | Sends a segment with SYN = 1, ACK = 1, Ack = x + 1, and its own sequence number (Seq = y). | Server acknowledges the client and sends its own request. |
| 3. ACK | Sends ACK = 1, Ack = y + 1, confirming receipt of server’s SYN. | Connection established. | Both sides are now synchronized and ready for data transfer. |

Diagram: TCP Three-Way Handshake

Client (Sender)                                Server (Receiver)

---------------------------------------------------------------

SYN, Seq = x  ---------------------------->   (Step 1)

<--------------------   SYN + ACK, Seq = y, Ack = x + 1  (Step 2)

ACK, Seq = x + 1, Ack = y + 1  ------------>   (Step 3)

---------------------------------------------------------------

Connection Established

2. Connection Termination – Four-Way Handshake

After communication is complete, TCP uses a four-way handshake to terminate the connection gracefully. This process ensures both sides have finished transmitting data and that all resources are released properly.

Steps Involved

| Step | Sender (Client) | Receiver (Server) | Description |
| --- | --- | --- | --- |
| 1. FIN | Sends FIN = 1, Seq = u, indicating no more data to send. | Waits for a termination request. | Initiates connection termination. |
| 2. ACK | Waits for acknowledgment. | Sends ACK = 1, Ack = u + 1. | Acknowledges the termination request. |
| 3. FIN | Waits for server’s FIN. | Sends FIN = 1, Seq = v, indicating it has no more data to send. | Server initiates its own termination. |
| 4. ACK | Sends ACK = 1, Ack = v + 1. | Connection closed. | Confirms termination and releases resources. |

Diagram: TCP Four-Way Handshake

Client (Sender)                                Server (Receiver)

---------------------------------------------------------------

FIN, Seq = u  ------------------------------->   (Step 1)

<-----------------------  ACK, Ack = u + 1  (Step 2)

<-----------------------  FIN, Seq = v  (Step 3)

ACK, Ack = v + 1  ----------------------------->   (Step 4)

---------------------------------------------------------------

Connection Terminated

Key Points

- The three-way handshake establishes a reliable, synchronized, and full-duplex connection between two devices.

- The four-way handshake ensures that both sides terminate the connection independently after confirming all data has been received.

- After termination, TCP waits in the TIME-WAIT state to ensure delayed packets are cleared before fully closing the connection.

Conclusion

TCP connection establishment and termination are essential for ensuring reliable, ordered, and error-free data transmission. The three-way handshake synchronizes communication parameters, while the four-way handshake gracefully closes the connection, maintaining data integrity and preventing loss.