1) Describe the role of the Application Layer in the OSI model. Explain HTTP and FTP protocols in detail.

Introduction to Application Layer

- The Application Layer is the 7th and topmost layer of the OSI (Open Systems Interconnection) model.

- It provides the interface between end-user applications and the underlying network services.

- While lower layers deal with data delivery and transmission, the application layer provides services and functionalities that are directly visible to users.

- It enables communication between software applications on different systems using well-defined protocols.

Functions of Application Layer

- Providing Network Services to Users

  - Examples: Email, web browsing, file transfer, remote login.

- Resource Sharing

  - Allows access to distributed resources like files, printers, and databases.

- Directory Services

  - Helps in identifying and locating resources on the network.

- Interoperability

  - Ensures that applications developed on different systems can communicate.

- User Authentication and Security

  - Provides login, password, and encryption mechanisms.

HTTP (HyperText Transfer Protocol)

- HTTP is the foundation of the World Wide Web (WWW).

- Type: Application Layer protocol.

- Underlying Protocol: Works on TCP/IP (usually TCP port 80, HTTPS on port 443).

- Nature: Stateless protocol, meaning each request is independent and server does not remember previous requests.

Working of HTTP

- A client (web browser) sends an HTTP request to a server.

- The server processes the request and sends an HTTP response back.

- The browser interprets the response and displays it to the user.

HTTP Request Format

GET /index.html HTTP/1.1

Host: www.example.com

User-Agent: Chrome/117.0

Accept: text/html

HTTP Response Format

HTTP/1.1 200 OK

Date: Mon, 24 Aug 2025

Content-Type: text/html

Content-Length: 512

<html> ... </html>

HTTP Methods

- GET – Retrieve a resource.

- POST – Send data (e.g., login form).

- PUT – Update resource.

- DELETE – Remove resource.

- HEAD – Retrieve headers only.

Advantages

- Simple, widely supported, flexible.

Limitations

- Stateless → requires cookies or sessions for state.

- Unsecure unless HTTPS is used.

FTP (File Transfer Protocol)

- FTP is a standard application protocol for transferring files between computers over TCP/IP.

- Ports: Control connection → TCP 21, Data connection → TCP 20.

- Model: Client-Server.

Working of FTP

- Client initiates connection to FTP server using control connection (port 21).

- Authentication: Username & Password.

- Server opens a data connection (port 20) to transfer files.

- Client can upload (put), download (get), rename, or delete files.

Modes of FTP

- Active Mode

  - Server initiates data connection.

- Passive Mode

  - Client initiates data connection (common behind firewalls).

Example FTP Session

ftp> open ftp.example.com

Connected to ftp.example.com

Name: student

Password: *****

ftp> get notes.pdf

ftp> put project.zip

ftp> quit

Conclusion

- Application Layer protocols are crucial for end-user communication.

- HTTP is widely used for web communication, while FTP is essential for file transfer.

- Together, they make the Internet user-friendly and efficient.

===========================================================================

2) What is DNS? Explain its purpose and working with an example.

Introduction

- DNS = Domain Name System.

- It is the naming system of the Internet, similar to a phonebook.

- Translates human-readable names (domain names) into machine-readable IP addresses.

Purpose of DNS

- User Convenience

  - Instead of remembering 142.250.182.4, user can type www.google.com.

- Scalability

  - Provides a distributed database, so no single server overloads.

- Load Balancing

  - A single domain name can map to multiple IPs → helps distribute traffic.

- Security

  - Supports DNSSEC for authentication of responses.

DNS Hierarchy

- Root DNS Servers – Top-most servers.

- TLD (Top Level Domain) Servers – Handle domains like .com, .org, .in.

- Authoritative DNS Servers – Contain IP addresses of specific domains.

Working of DNS (Step-by-Step)

Example: User wants to open www.openai.com.

- User enters domain in browser.

- Request goes to DNS Resolver (usually provided by ISP).

- Resolver first checks local cache. If not found:

  - Queries Root DNS server → points to .com TLD server.

  - Queries .com TLD server → points to openai.com authoritative server.

  - Authoritative server → provides final IP (e.g., 104.18.6.195).

- Resolver returns IP to the browser.

- Browser connects to server using that IP.

Example in Real Life

- Domain: www.google.com

- Resolved IP: 142.250.182.4

- Without DNS, users would need to type 142.250.182.4 instead of www.google.com.

Conclusion

- DNS is a critical backbone service of the Internet.

- It ensures usability, scalability, and efficiency by translating names into IPs.

===========================================================================

3) Explain the major design issues of the Data Link Layer. Why are error detection and flow control important?

Introduction

- Data Link Layer = 2nd layer of OSI model.

- Responsible for node-to-node delivery of frames over a physical medium.

Major Design Issues

- Framing

  - Breaks raw bitstream into frames.

  - Methods: Character Count, Byte Stuffing, Bit Stuffing.

- Error Detection and Correction

  - Transmission errors occur due to noise and interference.

  - Detection: Parity, Checksum, CRC.

  - Correction: Automatic Repeat Request (ARQ), Forward Error Correction.

- Flow Control

  - Prevents sender from overwhelming receiver.

  - Stop-and-Wait and Sliding Window protocols used.

- Access Control (MAC)

  - When multiple devices share medium, collisions occur.

  - Controlled by CSMA/CD (Ethernet), CSMA/CA (Wi-Fi), Token Passing.

Importance of Error Detection

- Ensures data integrity.

- Detects corrupted frames and requests retransmission.

- Without error detection, data communication would be unreliable.

Importance of Flow Control

- Balances speed mismatch between sender and receiver.

- Prevents buffer overflow.

- Ensures smooth, reliable, and efficient communication.

Conclusion

- The Data Link Layer ensures reliable communication.

- Without error detection and flow control, network communication would fail.

===========================================================================

4) Describe the Stop-and-Wait Flow Control Protocol with the help of a diagram.

Introduction

- Stop-and-Wait is the simplest ARQ (Automatic Repeat Request) protocol.

- Used for flow control and error control.

Working

- Sender sends one frame at a time.

- Waits for ACK from receiver.

- If ACK received → sends next frame.

- If ACK lost → retransmits after timeout.

- Receiver discards duplicates.

Diagram

Sender                               Receiver

------------------------------------------------

Send Frame 0   --------------------> Receive Frame 0

Wait for ACK   <-------------------- Send ACK 0

Send Frame 1   --------------------> Receive Frame 1

Wait for ACK   <-------------------- Send ACK 1

Advantages

- Simple to implement.

- Reliable for small networks.

Disadvantages

- Inefficient in long-distance/high-speed networks → long waiting time.

- Low channel utilization.

Conclusion

- Stop-and-Wait ensures reliable delivery but is inefficient.

- Improved by Sliding Window protocols (Go-Back-N, Selective Repeat).

===========================================================================

5) Write notes on Wireless LAN.

Introduction

- Wireless LAN (WLAN) is a type of Local Area Network that uses radio waves instead of cables.

- Standardized by IEEE 802.11 family (Wi-Fi).

Characteristics

- Coverage area limited to 100–300 meters indoors.

- Uses Access Points (APs) and wireless NICs.

- Operates in 2.4 GHz, 5 GHz, and 6 GHz bands.

- Supports mobility, portability, and flexibility.

Components

- Access Point (AP): Connects wireless users to wired LAN.

- Wireless Clients: Smartphones, laptops, IoT devices.

- Distribution System: Interconnects multiple APs.

- Wireless NIC: Hardware in each device.

Advantages

- Easy installation (no cables).

- Mobility and scalability.

- Cost-effective for temporary setups.

Disadvantages

- Security issues (eavesdropping, hacking).

- Interference from microwave, Bluetooth, etc.

- Limited range and lower bandwidth than wired LAN.

Applications

- Educational institutions.

- Business and corporate offices.

- Public Wi-Fi hotspots (cafes, airports).

- Smart homes.

Conclusion

- WLAN provides mobility, flexibility, and convenience.

- Despite limitations, it is a backbone of modern networking.

- With Wi-Fi 6 and Wi-Fi 7, WLANs are becoming faster and more secure.

===========================================================================

6) Write short notes on any two of the following:

a) Error detection and correction methods

b) Sliding Window Protocols

c) HDLC protocol.

Introduction

In computer networks, reliable communication between two systems is the most critical requirement. The OSI reference model defines different layers, each with a specific role, and the Data Link Layer plays a key role in ensuring that data is transferred reliably and efficiently over the physical medium.

The Data Link Layer provides services such as:

- Framing: Grouping bits into recognizable units.

- Error Detection and Correction: Ensuring that data received is accurate.

- Flow Control: Preventing fast senders from overwhelming slow receivers.

- Access Control: Determining which device can use the channel at a given time.

To implement these services, various methods and protocols are designed:

- Error detection and correction methods ensure that transmitted data remains accurate despite noise or disturbances.

- Sliding Window Protocols implement flow and error control for reliable delivery.

- HDLC protocol is a standard bit-oriented protocol that combines these mechanisms for effective data link communication.

(a) Error Detection and Correction Methods

In data communication, transmission of data over a network is subject to various impairments such as noise, attenuation, distortion, and interference. These factors may introduce errors in the transmitted bits. Since accurate data delivery is essential, networks employ error detection and correction techniques.

1. Error Detection

Error detection methods add redundancy (extra bits) to the original data, enabling the receiver to check whether errors occurred during transmission.

- Parity Check:

  - A parity bit (even or odd) is appended to the data.

  - Example: For even parity, if data contains an odd number of 1’s, an additional 1 is added to make the total count even.

  - Simple but can only detect single-bit errors.

- Checksum:

  - Data is divided into equal-sized segments, and all segments are summed.

  - The sum is then complemented and sent with the data.

  - The receiver performs the same calculation to verify correctness.

  - Widely used in Internet protocols (TCP/UDP, IP).

- Cyclic Redundancy Check (CRC):

  - Uses polynomial division to generate a fixed-length check value.

  - The sender divides data bits by a predetermined divisor (generator polynomial) and appends the remainder as the CRC bits.

  - The receiver re-divides; if remainder ≠ 0, an error is detected.

  - Very reliable, widely used in Ethernet, HDLC, etc.

2. Error Correction

Detection only identifies errors; correction enables recovery of the original message without retransmission.

- Forward Error Correction (FEC):

  - Extra redundant bits allow the receiver to not only detect but also correct certain errors.

  - Example: Hamming Code (detects and corrects single-bit errors, detects double-bit errors).

  - Used in noisy communication systems like satellite links, wireless networks.

- Automatic Repeat Request (ARQ):

  - If an error is detected, the receiver requests retransmission.

  - Examples: Stop-and-Wait ARQ, Go-Back-N ARQ, Selective Repeat ARQ.

  - Ensures reliability but increases delay.

3. Importance

- Ensures reliable data communication.

- Provides robustness against channel noise.

- Essential for applications like bank transactions, emails, file transfer, multimedia streaming.

Conclusion: Error detection and correction are fundamental to network reliability. While detection methods like CRC are efficient, correction techniques like Hamming Codes or ARQ ensure recovery of lost information. Both together maintain data integrity across networks.

(b) Sliding Window Protocols

Sliding window protocols are flow control and error control mechanisms used at the Data Link and Transport layers. They manage how many frames can be sent before needing an acknowledgment, thus maximizing channel utilization.

1. Concept of Sliding Window

- A window of a fixed size defines the maximum number of outstanding (unacknowledged) frames allowed.

- Sender maintains a window of sequence numbers it can send.

- Receiver maintains a window of sequence numbers it can accept.

- As acknowledgments are received, the window “slides” forward, allowing transmission of new frames.

2. Types of Sliding Window Protocols

- Stop-and-Wait Protocol

  - Simplest form.

  - Sender transmits one frame and waits for acknowledgment before sending the next.

  - Simple but inefficient, especially on high-delay or high-bandwidth links.

- Go-Back-N ARQ

  - Sender can transmit multiple frames (window size > 1).

  - If an error occurs, all frames from the erroneous one onward are retransmitted.

  - Efficient but wastes bandwidth if errors are frequent.

- Selective Repeat ARQ

  - More advanced.

  - Receiver acknowledges correctly received frames even if they are out of order.

  - Sender retransmits only the erroneous frames, not the entire batch.

  - Efficient use of bandwidth but requires buffering and complexity at the receiver.

3. Advantages of Sliding Window Protocols

- Improve channel utilization compared to Stop-and-Wait.

- Enable pipelining of frames.

- Balance between throughput and reliability.

4. Applications

- Used in HDLC (High-Level Data Link Control).

- TCP uses a sliding window mechanism for reliable communication.

Conclusion: Sliding window protocols are the backbone of reliable, efficient communication. By combining flow control and error handling, they ensure optimal usage of the transmission medium and are widely applied in modern networking protocols.

(c) HDLC Protocol

HDLC (High-Level Data Link Control) is an ISO standard bit-oriented data link layer protocol. It provides framing, error detection, flow control, and reliable communication over point-to-point and multipoint links.

1. Characteristics

- Bit-oriented: Uses bit patterns for control instead of characters.

- Provides error detection (CRC) and error recovery (ARQ).

- Supports both connection-oriented and connectionless services.

2. Frame Structure

HDLC uses structured frames to ensure proper communication. A typical frame contains:

- Flag Field (01111110) – Marks the beginning and end of a frame.

- Address Field – Identifies the secondary station.

- Control Field – Specifies frame type and sequence numbers.

- Information Field – Contains user data (payload).

- FCS (Frame Check Sequence) – CRC used for error detection.

3. Types of Frames

- Information Frames (I-frames): Carry user data and control information.

- Supervisory Frames (S-frames): Provide flow and error control (ACK, NAK, etc.).

- Unnumbered Frames (U-frames): Used for control and management (link establishment, disconnection, etc.).

4. Modes of Operation

- Normal Response Mode (NRM): Secondary station transmits only when permitted by the primary station.

- Asynchronous Balanced Mode (ABM): Both stations can initiate communication independently (peer-to-peer).

- Asynchronous Response Mode (ARM): A secondary can transmit without explicit permission but still under control of primary.

5. Applications

- Widely used in WAN technologies (e.g., ISDN, X.25).

- Basis for many other data link protocols (PPP, LAPB).

Conclusion: HDLC is a robust protocol ensuring framing, flow control, and error detection. Its versatility and reliability made it the foundation for several modern data link protocols.