# Routing in the Network Layer

## 1. Main Function of the Network Layer

- The network layer is responsible for delivering packets from the source machine to the destination machine.

- In most networks, packets must travel across multiple hops (routers) before reaching the destination.

- Broadcast networks are the only major exception, but even here routing is needed if the sender and receiver are on different segments.

## 2. Routing and Forwarding

- Routing = The decision-making process to determine which route (path) a packet should take.

- Forwarding = The actual action of sending a packet to the correct outgoing line based on the routing table.

A router has two processes:

- Forwarding process → Handles each packet arrival, looks up the routing table, and forwards.

- Routing process (algorithm) → Builds and updates the routing tables.

## 3. Datagram vs Virtual Circuit Routing

- Datagram networks: Routing decisions are made for every packet (may change each time).

- Virtual-circuit networks: Routing decision is made once when the circuit is set up, and packets follow the fixed path.

  - Also called session routing (route remains valid for an entire session, e.g., VPN connection).

## 4. Desired Properties of Routing Algorithms

- Correctness – Should always find valid routes.

- Simplicity – Algorithm should not be overly complex.

- Robustness – Must tolerate failures of hosts, routers, and links without stopping the network.

- Stability – Should converge to a fixed set of routes (avoid endless oscillation).

  - Must converge quickly to minimize disruptions.

- Fairness – Each user/connection gets a reasonable share of network resources.

- Efficiency – Optimizes the use of network resources.

## 5. Optimization Goals

- Different criteria for “best” routing:

  - Minimize mean packet delay

  - Maximize total throughput

  - Minimize hop count or travel distance

- Conflict: High throughput often increases queueing delay.

- Practical compromise → minimize hops → improves both delay and bandwidth usage.

## 6. Types of Routing Algorithms

### (A) Nonadoptive Routing (Static)

- Routes are precomputed offline and loaded when the network starts.

- Do not change with traffic or failures.

- Useful when the best route is obvious.

  - Example: Router F always forwards packets to router E.

### (B) Adaptive Routing (Dynamic)

- Routes change dynamically based on topology and sometimes traffic conditions.

- Differ in:

  - Information source: local (router itself), neighbours, or global (all routers).

  - Update timing: on topology changes or periodically (every ΔT seconds).

  - Optimization metric: hop count, distance, estimated transit time, etc.

## 7. Broader Routing Scenarios

- Not limited to single-source-to-single-destination.

- Can include:

  - Multicast (one-to-many),

  - Broadcast (one-to-all),

  - Anycast (one-to-one-of-many).