## Approaches to Congestion Control in the Network Layer

### 1. Introduction

Congestion occurs in a network when the demand for resources (such as bandwidth or buffers) exceeds the available capacity, leading to performance degradation—like increased delay, packet loss, or even network collapse.
To handle this, networks employ congestion control mechanisms—techniques to prevent, detect, and manage congestion.

Tanenbaum classifies congestion control methods broadly into two categories:

- Open-loop congestion control (prevention):
Designed into the network to prevent congestion before it occurs.
These are proactive methods and work by careful resource management and routing.

- Closed-loop congestion control (reaction):
These mechanisms detect congestion and react to it dynamically by adjusting transmission rates or network parameters.
These are reactive methods.

## 2. Open-Loop (Preventive) Approaches

Open-loop mechanisms act before congestion happens, by controlling parameters such as routing, traffic entry, and load balancing.

### (a) Traffic-Aware Routing

- Idea: Avoid congested parts of the network when selecting routes.

- Explanation:
In traffic-aware routing, routers continuously monitor network traffic and select paths that are less loaded.
For instance, if a router notices that a particular link is heavily congested, it may redirect packets through an alternate, less congested path.

- Example:
In adaptive routing, metrics such as queue length, delay, or available bandwidth are used to dynamically adjust routes.

- Advantage: Reduces the likelihood of congestion buildup in specific links.

### (b) Admission Control

- Idea: Prevent new traffic from entering the network when it is already heavily loaded.

- Explanation:
When a new virtual circuit or flow requests to enter the network, the router first checks whether sufficient resources (bandwidth, buffers) are available along the path.
If not, the connection is denied or delayed until congestion subsides.

- Example:
Used widely in networks providing Quality of Service (QoS), such as ATM and MPLS networks.

- Purpose: Ensures that the existing connections get guaranteed performance without overload.

### (c) Traffic Throttling (Load Regulation)

- Idea: Limit the rate at which sources inject packets into the network.

- Explanation:
When early signs of congestion are detected, the network can signal sources to slow down their transmission rate.
This can be implemented using:

  - Leaky bucket or token bucket algorithms to smooth traffic bursts.

  - Explicit congestion notification (ECN) messages from routers.

- Benefit: Prevents queues from overflowing and stabilizes traffic flow.

### (d) Load Shedding

- Idea: Drop packets selectively when congestion becomes severe.

- Explanation:
When buffers in routers or switches overflow, the system must discard packets.
Load shedding policies decide which packets to drop first, e.g.:

  - Drop low-priority packets first.

  - Drop random packets to maintain fairness (used in Random Early Detection – RED).

- Purpose:
To reduce the load quickly, allowing the system to recover from congestion.

## 3. Closed-Loop (Reactive) Approaches

While the above open-loop mechanisms try to prevent congestion, closed-loop methods detect and correct it after it happens.
They typically involve three phases:

- Monitoring the system to detect congestion.

- Passing information about congestion to the sources.

- Adjusting transmission rates or routing decisions.

Examples include:

- Choke packets: Routers send choke packets back to the source to signal congestion.

- Hop-by-hop backpressure: Routers notify upstream neighbours to slow down.

- Explicit Congestion Notification (ECN): Routers mark packets instead of dropping them to indicate congestion to endpoints.

## 4. Summary Table

| Approach | Type | Description | Goal |
| --- | --- | --- | --- |
| Traffic-Aware Routing | Open-loop | Avoids congested paths dynamically | Prevent congestion |
| Admission Control | Open-loop | Refuses new flows when capacity is low | Prevent overload |
| Traffic Throttling | Open/Closed-loop | Limits packet injection rate | Maintain stability |
| Load Shedding | Closed-loop | Drops packets when buffers overflow | Relieve congestion |

- Congestion control is essential to maintain network performance and stability.

- Open-loop methods (like admission control, routing) aim to prevent congestion.

- Closed-loop methods (like choke packets, ECN) aim to respond to congestion.

- A combination of both is used in modern networks such as the Internet to ensure efficiency and fairness.