# INTERNETWORKING – Summary

## 1️.Concept

Internetworking refers to connecting multiple heterogeneous networks (LANs, WANs, MANs, etc.) to enable communication among hosts on different networks. The best-known example is the Internet — a global internetwork. Larger networks are more valuable (Metcalfe’s Law: Value ∝ N²). However, networks differ in technology, addressing, and services, creating interoperability challenges.

## 2️.How Networks Differ

| Aspect | Possible Differences |
| --- | --- |
| Service | Connection-oriented vs. connectionless |
| Addressing | Flat or hierarchical; different lengths |
| Broadcasting | May or may not support broadcast/multicast |
| Packet size | Varying maximum frame sizes (MTU) |
| Ordering | Ordered or unordered delivery |
| Quality of Service (QoS) | Guaranteed or best-effort |
| Reliability/Security | Different error control, encryption, privacy rules |
| Accounting | By time, byte, packet, or none |

Routers and gateways must handle fragmentation, reassembly, buffering, and protocol translation to overcome these differences.

## 3️.How Networks Are Connected

Two approaches are used to connect networks:
1. Translation – Converting packet formats (complex, rarely used).
2. Common Layer – Using a universal internetworking layer (IP) to hide differences.

The TCP/IP model (Cerf & Kahn, 1974) uses IP as the common layer. Routers operate at the network layer and forward packets based on IP addresses, while switches/bridges work at the link layer using MAC addresses. Internetworking works only if a common network-layer protocol (IP) is shared.

## 4️. Tunneling

Tunneling allows packets of one protocol to travel through another network by encapsulation. Example: IPv6 over IPv4. The IPv6 packet is wrapped inside an IPv4 header forming a tunnel, with only the border routers understanding both protocols. Tunneling is widely used for VPNs and overlay networks.

## 5️. Internetwork Routing

Routing across interconnected networks is complex because different networks may use distinct routing algorithms and policies. Business, cost, and privacy issues further complicate path selection.

To manage this, a two-level routing structure is used:
1. Intradomain (IGP) – Within an Autonomous System (AS), e.g., OSPF, RIP.
2. Interdomain (EGP) – Between ASes, using BGP (Border Gateway Protocol).

Each AS operates independently but follows routing policies to ensure scalable and secure routing across the Internet.

## 6️. Packet Fragmentation

Each network has a Maximum Transmission Unit (MTU), defining the largest packet it can carry. Large packets are fragmented to cross smaller-MTU links and reassembled at the destination. Common MTUs include Ethernet (1500 bytes) and IEEE 802.11 (2272 bytes).

## Key Takeaway

Internetworking succeeds through:
• A common IP layer across diverse networks
• Efficient routing (BGP + IGP)
• Tunneling for protocol compatibility
• Fragmentation for handling MTU differences

Together, these enable seamless global Internet connectivity.