## INTERNET PROTOCOL (IP)

### 1. Introduction

The Internet Protocol (IP) is the core protocol of the network layer in the TCP/IP suite.
It provides the basic mechanism for delivering packets (called datagrams) from the source to the destination across interconnected networks—without guaranteeing delivery, order, or duplicate protection.

In simple terms: IP provides the best-effort, connectionless delivery of packets across different networks.

Tanenbaum describes IP as the “glue that holds the Internet together.”
Every device on the Internet must have an IP address, and IP ensures that packets find their way across multiple networks to reach the destination.

### 2. Characteristics of IP

| Characteristic | Description |
| --- | --- |
| Connectionless Service | IP does not establish a connection before sending data. Each packet is treated independently. |
| Unreliable Service | IP does not guarantee packet delivery, order, or duplication control. Reliability is handled by higher layers (e.g., TCP). |
| Best-Effort Delivery | IP tries its best to deliver packets but may drop them in case of congestion or errors. |
| Host Addressing | Each interface in a network has a unique IP address (IPv4 or IPv6). |
| Routing | IP determines how packets move through routers from source to destination. |
| Fragmentation & Reassembly | Large packets may be split (fragmented) for transmission and reassembled at the destination. |

### 3. Structure of an IP Datagram (IPv4)

An IP datagram consists of two parts:

- Header – contains control information used for routing and delivery.

- Data – the payload from upper layers (e.g., TCP, UDP).

### 4. IPv4 Datagram Format

| Field | Size (bits) | Description |
| --- | --- | --- |
| Version | 4 | Indicates IP version (4 for IPv4). |
| Header Length (IHL) | 4 | Length of header in 32-bit words. |
| Type of Service (TOS) | 8 | Specifies priority and handling of packets (now replaced by DSCP). |
| Total Length | 16 | Total size of the datagram (header + data). |
| Identification | 16 | Unique number to identify fragments of the same datagram. |
| Flags | 3 | Control fragmentation (e.g., Don't Fragment). |
| Fragment Offset | 13 | Position of this fragment in the original datagram. |
| Time to Live (TTL) | 8 | Maximum hops allowed before the packet is discarded. Prevents infinite looping. |
| Protocol | 8 | Indicates the upper-layer protocol (e.g., TCP=6, UDP=17, ICMP=1). |
| Header Checksum | 16 | Error checking for the header. |
| Source Address | 32 | Sender’s IP address. |
| Destination Address | 32 | Receiver’s IP address. |
| Options (if any) | Variable | Used for testing, debugging, or routing. |
| Data | Variable | Actual payload. |

### 5. Working of IP

- Encapsulation:
The IP layer encapsulates the data received from the transport layer (TCP/UDP) into a datagram.

- Addressing:
It assigns the source and destination IP addresses.

- Routing:
Each router along the path examines the destination address, looks up its routing table, and forwards the packet to the next hop.

- Fragmentation:
If the datagram is larger than the network’s MTU (Maximum Transmission Unit), IP fragments it into smaller packets.

- Delivery:
The packet is delivered to the destination host, where the network layer reassembles any fragments and delivers the data to the transport layer.

### 6. Functions of IP

| Function | Description |
| --- | --- |
| Addressing | Assigning unique IP addresses to devices. |
| Routing | Selecting the best path to the destination. |
| Fragmentation & Reassembly | Splitting large datagrams and reassembling them. |
| Error Handling | Limited error detection through header checksum. |
| Encapsulation | Adding IP headers to transport-layer segments. |

### 7. IP at the Network Layer in TCP/IP Model

| Layer | Role | Example Protocols |
| --- | --- | --- |
| Application Layer | Provides network services | HTTP, DNS, SMTP |
| Transport Layer | Ensures end-to-end communication | TCP, UDP |
| Network Layer | Routes packets across networks | IP (IPv4/IPv6), ICMP, ARP |
| Data Link Layer | Handles local transmission | Ethernet, PPP |
| Physical Layer | Transmits raw bits | Cables, wireless signals |

### 8. Supporting Protocols

Tanenbaum also mentions that IP works alongside several support protocols:

- ICMP (Internet Control Message Protocol): Used for error reporting and diagnostics (e.g., ping).

- ARP (Address Resolution Protocol): Maps IP addresses to MAC addresses.

- RARP (Reverse ARP): Maps MAC addresses to IP addresses (obsolete).

- IGMP (Internet Group Management Protocol): Manages multicast group membership.

### 10. Summary (as per Tanenbaum)

| Aspect | Description |
| --- | --- |
| Protocol Type | Network layer, connectionless |
| Service Type | Best-effort delivery |
| Address Size | 32 bits (IPv4), 128 bits (IPv6) |
| Error Control | Header checksum (IPv4) |
| Fragmentation | Supported (IPv4), simplified in IPv6 |
| Routing | Hop-by-hop decision at routers |
| Reliability | Provided by upper layers (TCP) |

### In Essence

IP is the heart of the Internet.
It defines how packets are addressed, routed, and transmitted from one network to another, making global communication possible.
While IPv4 remains dominant, IPv6 represents the future of scalable and efficient internetworking.