A sliding window protocol is a network protocol that enables a sender to transmit multiple data frames without waiting for an acknowledgment for each one, using a "window" of a fixed or variable size to manage the number of unacknowledged frames in transit. It improves efficiency by keeping the communication channel busy, which is particularly beneficial for networks with high latency, and uses sequence numbers to ensure data is received in the correct order and to detect lost or duplicated frames.

How it Works

- 1. Sender Window:

The sender maintains a window representing frames that have been sent but not yet acknowledged.

- 2. Window Size:

The size of this window dictates how many frames the sender can transmit at a time without needing an acknowledgment for the previous one.

- 3. Transmission:

When the sender receives an acknowledgment (ACK) for a frame, its window "slides" forward, allowing it to send the next frame in the sequence.

- 4. Sequence Numbers:

Each frame is assigned a unique sequence number, which the receiver uses to identify missing frames and reassemble them in the correct order.

- 5. Receiver Window:

The receiver also maintains a window to track the expected sequence numbers of incoming frames and to store frames that arrive out of order.

Key Advantages

- Improved Efficiency:

By allowing multiple frames to be in transit, it prevents the sender from having to wait for acknowledgments after every single frame, leading to better bandwidth utilization and reduced waiting times.

- Reliable Data Transfer:

The use of sequence numbers and acknowledgments ensures that data is delivered reliably, even over unreliable network links where packets might be lost, duplicated, or reordered.