# A Protocol Using Go-Back-N

## 1. Introduction

Go-Back-N ARQ (Automatic Repeat reQuest) is a sliding window protocol used at the data link layer for reliable transmission. It allows the sender to transmit multiple frames before needing an acknowledgment. The receiver is simpler than the sender: it only keeps track of the next expected frame.

## 2. Basic Operation

Sender Side:
- Maintains a window of size N.
- Can send up to N frames without waiting for ACK.
- Must keep copies of all transmitted but not yet acknowledged frames.
- Uses a timer for the oldest unacknowledged frame.

Receiver Side:
- Maintains a variable Expected Frame.
- If the correct frame arrives: it is accepted and delivered to the network layer, and an ACK is sent.
- If an out-of-order frame arrives: it is discarded, and the last ACK is resent.

## 3. Acknowledgments

ACKs are cumulative. An ACK(n) means all frames up to n have been correctly received.
Example:
- If frames 0,1,2,3,4 are sent and the receiver sends ACK(3), the sender knows frames 0,1,2 have been received.

## 4. Error Handling (Go-Back-N Mechanism)

- If a frame is lost or corrupted: the receiver discards it and all subsequent frames, and keeps sending ACK for the last correctly received frame.
- When the sender’s timer expires, it goes back and retransmits the lost frame and all frames after it in the window.

## 5. Timing Diagram Example

Sender                         Receiver
Frame0 ----->                   
Frame1 ----->                   
Frame2 ----->  (Frame2 lost)   
Frame3 ----->                   
                 X  (discard 3, waiting for 2)
Timeout for 2
Resend Frame2 ----->           
Resend Frame3 ----->

## 6. Features

✔ Higher efficiency than Stop-and-Wait (multiple frames in transit).
✔ Works well in long-delay links.

## 7. Limitations

✘ If one frame is lost, many frames must be retransmitted (wastage).
✘ Receiver buffer is underutilized (cannot store out-of-order frames).

## 8. Applications

• Used in protocols like:
  - HDLC (High-level Data Link Control)
  - TCP (early implementations before selective acknowledgment)

## Summary

Go-Back-N ARQ improves efficiency by allowing multiple outstanding frames, but retransmits all frames after a lost one, making it less efficient than Selective Repeat ARQ in high-error environments.