# A Protocol Using Selective Repeat

## 1. Introduction

Selective Repeat ARQ is an advanced sliding window protocol for reliable transmission. Unlike Go-Back-N, where all frames after a lost one are resent, Selective Repeat only retransmits the specific frames that are lost or corrupted. It provides higher efficiency, especially in high-delay or error-prone networks.

## 2. Basic Operation

Sender Side:
- Maintains a window of size N.
- Can send multiple frames without waiting for ACKs.
- Keeps copies of unacknowledged frames in a buffer.
- Uses a timer for each outstanding frame.

Receiver Side:
- Maintains a window of acceptable sequence numbers.
- Can accept frames out of order and buffer them.
- Delivers frames to the network layer only when all previous frames have been received.
- Sends individual ACKs for each correctly received frame.

## 3. Acknowledgments

ACKs are individual, not cumulative.
Example:
- If frames 0,1,2,3,4 are sent and frame 2 is lost, ACKs for 0,1,3,4 are still sent.
- The sender retransmits only frame 2 when its timer expires.

## 4. Error Handling (Selective Repeat Mechanism)

- If a frame is lost or corrupted: the receiver buffers later frames instead of discarding them.
- It may send a NAK (negative acknowledgment) or remain silent for the missing frame.
- When the sender’s timer for that frame expires, it retransmits only that frame.

## 5. Timing Diagram Example

Sender                         Receiver
Frame0 ----->   ✔               
Frame1 ----->   ✔               
Frame2 ----->   (lost)          
Frame3 ----->   ✔  (stored, waiting for 2)
Frame4 ----->   ✔  (stored, waiting for 2)

Timeout for 2
Resend Frame2 ----->  ✔  (Now deliver 2,3,4 in order)

## 6. Features

✔ More efficient than Go-Back-N (only erroneous frames retransmitted).
✔ Receiver buffers out-of-order frames.
✔ Best suited for high-bandwidth and long-delay links.

## 7. Limitations

✘ More complex (requires more memory and processing at the receiver).
✘ Needs a separate timer for each frame.
✘ Requires larger sequence number space (at least 2N, where N = window size).

## 8. Applications

• Used in:
  - High-speed networks.
  - Reliable data transfer protocols where efficiency is critical.
  - Modern enhancements of TCP with Selective Acknowledgment (SACK) option.

## Summary

The Selective Repeat ARQ protocol improves reliability and efficiency by retransmitting only the erroneous frames, unlike Go-Back-N. Although more complex, it achieves much better performance on error-prone and long-delay channels.