# HDLC Protocol and Bit Stuffing

High-level Data Link Control (HDLC) is a bit-oriented protocol for communication

over point-to-point and multipoint links. It implements the Stop-and-Wait protocol.

It is the basis of many protocols in practice today. HDLC defines three types of frames:

Information frames,

Supervisory frames,

Unnumbered frames.

-The informational frames are used to carry data frames.

-Supervisory frames are used only to transport control information for flow and error control.

-Unnumbered frames are reserved for system management and provide connection-oriented service.

One of the most common protocols for point-to-point access is the Point-to-Point

Protocol (PPP). PPP uses only one type of frame, but allows multiplexing of different

payloads to achieve a kind of connection-oriented service authentication. Encapsulating

different packets in a frame allows PPP to move to different states to provide necessary

services.

## 1. High-Level Data Link Control (HDLC)

HDLC is a bit-oriented data link control protocol standardized by ISO. It is widely used in both point-to-point and multipoint communication. Each frame begins and ends with a special flag pattern.

## 2. Flag Pattern

The flag is a unique 8-bit pattern:
01111110 (in binary) = 0x7E (in hexadecimal).
Flags mark the boundaries of frames: one flag at the start and another at the end.

## 3. Problem with Flag Pattern in Data

The flag pattern (01111110) might accidentally appear in the user data. If this happens, the receiver may mistakenly interpret part of the data as a frame boundary. To avoid this, bit stuffing is used.

## 4. Bit Stuffing

Definition: The process of inserting extra 0s into the data stream to ensure that the flag sequence never appears inside the data.
Rule: Whenever the sender encounters five consecutive 1s in the data, it inserts a 0 bit immediately after them.

Example:
Data to send: 01111110
Sent (after bit stuffing): 011111010
This ensures that the actual flag (01111110) is not confused with data.

## 5. Destuffing at Receiver

The receiver continuously monitors the incoming stream. Whenever it sees five consecutive 1s followed by a 0, it removes (destuffs) the 0. This restores the original data exactly as it was before transmission.

## 6. Transparency to Higher Layers

Bit stuffing is completely transparent to the network layer: Sender’s data are delivered unchanged to the receiver. The process of stuffing and destuffing happens only at the data link layer.

## 7. Synchronization Benefit

Bit stuffing ensures minimum density of transitions (0s and 1s alternate enough). This helps the physical layer maintain synchronization between sender and receiver.

## 8. Example (Bit Stuffing in Action)

Suppose user data contains:
01111110
Sender (after stuffing): 011111010
Receiver (after destuffing): 01111110

Thus, the flag pattern cannot occur within the data, only at frame boundaries.

## 9. Applications of Bit Stuffing

Bit stuffing is used in:
- HDLC (High-level Data Link Control)
- USB (Universal Serial Bus) for synchronization purposes.

Key Point: Bit stuffing ensures that the flag pattern (01111110) is used only as a frame delimiter and never occurs inside user data, enabling error-free frame boundary recognition.